Nursery Landscape Problem Solving

Mark answers in the Assessment and Solution Section on score card.

| Figure 1. American Standard for Dogwood Rootball |  |  |
| --- | --- | --- |
| Up to 6 feet Height | Over 6 feet Caliper | Minimum Diameter Ball(inches) |
| 2 |  | 10 |
| 3 |  | 12 |
| 4 |  | 14 |
| 5 |  | 16 |
|  | 3/4" | 16 |
|  | 1" | 18 |
|  | 11/2" | 20 |
|  | 13/4" | 22 |
|  | 2" | 24 |
|  | 21/2" | 28 |
|  | 3" | 32 |
|  | 31/2" | 38 |
|  | 4 | 42 |
|  | 41/2" | 48 |
|  | 5" | 54 |

Ball Depth Ratio

Diameter less than 20"

Depth not less than 75%

of diameter or 3/4 of width

Diameter 20"-30"

Depth not less

than 66 2/3%

or 2/3 of width

##### Diameter 31"-48"

Depth not less than 60%

or 3/5 width. Balls with

a diameter of 30" or more

should be drum-laced

Question

Given the above information, what minimum depth should the root ball for a 3” caliper field grown dogwood tree be dug?

Select from the below answers:

## A.   12 inches						B. 19 inches

C.  28 inches						D. 32 inches

Answer: B. 19 inches

Solution:

1. 3” caliper field grown dogwood tree minimum diameter of root ball = 32”

2. Ball Depth Ratio  Diameter 31-48"  = Depth not less than 60% of diameter or 3/5 of width

32 x .60 = 19” deep as minimum depth dug