Student:________________________

Class period_____________________

Procedure Check-list for Drafting the Floor Plan

Description of steps					           Points earned	 Date

| Step 1 | Lay out the exterior walls making sure the plan is centered on paper. Use construction lines only and be sure to use 1/4" =1'-0 scale. | /10 |  |
| --- | --- | --- | --- |
| Step 2 | Locate and draw the interior walls using construction lines only. Make sure walls are 6" wide and that room sizes meet standards. | /10 |  |
| Step 3 | Determine the location and size of doors and windows and draw on plan. Use architectural template for door symbols. These symbols should be drawn as object lines. | /10 |  |
| Step 4 | Draw the stairs if interior stairs are included in the floor plan. | /10 |  |
| Step 5 | Locate and draw the fireplace if included in the floor plan. | /10 |  |
| Step 6 | Locate and draw walks, patios and/or decks. | /10 |  |
| Step 7 | Draw the kitchen cabinets, appliances and bathroom fixtures. Use an architectural template for drawing plumbing fixtures and standard appliances. | /10 |  |
| Step 8 | Add dimensions, notes and room names. (Text should be 1/8" high and ALWAYS USE GUIDELINES!) Be sure to use proper dimension techniques. | /20 |  |
| Step 9 | Add material and identification symbols. Darken lines and remember all lines should be dark and only vary in thickness with the exception of construction and guide lines. Keep hidden and center lines thin. | /20 |  |
| Step 10 | Draw the border, title block and add the scale. (1/4"=1'-0) | /10 |  |
| Step 11 | Check the entire drawing. Then have a peer check the drawing. |  |  |

This checklist should be used in determining how to draft a floor plan.  It is the student’s responsibility to have each step checked by the instructor.  This checklist is to be turned in at the time of completion of the floor plan.