- What do floor plans show?

- all items to be permanently built

- dimensions

- plumbing fixtures

- all of the above

- When drawing a floor plan, what is drawn first?

- exterior walls, cabinets, dimensions

- exterior walls, interior walls, door and window openings

- interior walls, cabinets, exterior walls

- exterior walls, door and window openings, cabinets

- At what scale is the floor plan typically drawn?

- 1/4" = 1'-0"

- 3/32" = 1'-0"

- 1" = 10'

- none of the above

- When showing built-in items above the 4' line, what line type is used?

- center

- continuous

- dashed

- none of the above

- Plumbing walls are drawn at what thickness?

- 2x4

- 2x6

- 2x8

- 2x12

- At what thickness are exterior walls shown?

- 6”

- 8”

- depends on the material

- none of the above

- Are doors and windows called out on a floor plan?

- No, only on an elevation.

- Yes, they are identified with a letter or number for an attached schedule.

- Yes, only the size is important.

- None of the above

- Are objects that are not seen, such as an oven, shown on the floor plan?

- Yes, with a hidden line.

- Sometimes, with a continuous line.

- Never.

- Only if they are an important feature.

- Are room labels important?

- No, they make no difference.

- Sometimes

- Yes, they are important to coordinate locations with other plans.

- None of the above

- Why is the use of poche on walls helpful?

- It helps the walls to easily be distinguishable from other floor plan objects.

- Different size and types of walls can be shown differently.

- Both a and b are correct.

- None of the above

- Why are symbols used on the floor plan?

- to describe items that are associated with the home

- to brighten the plan up a little for presentation purposes

- a and b

- none of the above

- Why is it important to keep symbol sizes to scale?

- There will be many symbols, notes, and dimensions.

- To keep the plan uncluttered.

- It is not important, any size will do.

- None of the above

- Where does the drafter get the symbols for use?

- The drafter makes up his/her own symbols.

- If drawing with CADD, there is a symbol library.

- Reference materials.

- Both b and c

- What type symbols are used when drawing wall sections?

- 2x4 stud is a rectangle with an x in it

- concrete hatching in a footing

- no symbols used in wall sections

- a and b

- What type symbol is used on a floor plan for walls?

- none

- wall shading

- thick lines for the outline only

- none of the above

- Are all door symbols the same?

- No, there is a different symbol for each type of door.

- All doors are shown the same and noted differently.

- The drafter uses whatever symbols he/she wants.

- None of the above.

- How are door and windows called out?

- with a circle or a hexagon with a number or a letter in it

- with a circle only

- with a hexagon only

- with a square

- What symbols are shown in a kitchen?

- appliances

- countertops

- plumbing fixtures

- all of the above

- What symbol is not shown in a bathroom?

- bathtub

- lavatory

- water closet

- cantilever

- What symbol is used to show directions of stairs and slopes of roofs?

- square with the direction noted

- arrow pointing in the right direction

- a circle stating north, south, east, or west

- none of the above

- If a kitchen design contains an island, what is the base clearance around it?

- 4'-0"

- 3'-0"

- 2'-6"

- 5'-0"

- How deep is a base kitchen cabinet?

- 22"

- 20"

- 24"

- 30"

- When designing a kitchen, is it important to have natural light?

- always

- never

- sometimes

- none of the above

When designing a kitchen, what is taken into consideration?