- When describing a roof pitch, 8/12, what does the number 8 represent?

- the horizontal run in the angle of the roof

- the vertical rise in the angle of the roof

- a and b

- none of the above

- When describing a roof pitch 6/12, what does the number 12 represent?

- the horizontal run in the angle of the roof

- the vertical rise in the angle of the roof

- a and b

- none of the above

- How can a drafter draw the slope lines of the roof?

- by using the roof angle the pitch represents

- by measuring the rise and run of a roof

- a and b

- none of the above

- Where is the roof type and roof pitch shown?

- roof framing plan

- roof plan

- elevations

- all of the above

- A dutch hip roof is a combination of what two roof types?

- hip and gable

- gable and shed

- hip and flat

- A-frame and hip

- What is the difference between a hip roof and a gable roof?

- A gable roof has four surfaces and a hip roof has two.

- A hip roof has four surfaces and a gable has two.

- There is no difference between a hip roof and a gable roof.

- None of the above

- What is a common roof type in climates with little or no snow or rain?

- gable

- hip

- flat

- mansard

- What are three common roof shapes?

- gable, mansard, Dutch hip

- hip, mansard, Dutch hip

- flat, gable, gambrel

- gable, hip, flat

- What roof type is a traditional shape that dates back to the Colonial period?

- gambrel

- A-frame

- Dutch hip

- flat

- What is an opening framed in the roof to allow for window placement, most frequently used on a hip or gable roof?

- dormer

- eave

- alcove

- awning

- What objects are shown on a roof plan?

- roof overhang

- wall outline

- ridge lines

- all of the above

- What is formed at a 45-degree angle when two perpendicular roofs intersect each other?

- valley

- ridge

- hip

- none of the above

- What line type is used to show the outline of the walls below on the roof plan?

- hidden/dashed

- center line

- continuous

- none of the above

- Is the roof pitch and the direction of roof slope called out on the roof plan?

- always both

- only the roof pitch

- only the direction of roof slope

- neither

- After the outline of exterior walls and overhang limits are drawn, what is added next?

- notes

- ridges, hips, and valleys

- direction of roof slope

- skylights

- What is one of the uses of a roof plan?

- elevations

- foundation plan layout

- plumbing plan layout

- none of the above

- At what scale is the roof plan typically drawn?

- does not matter

- same as the floor plan

- 1"=10'-0"

- none of the above

- What lineweight is used for the roof lines?

- object - thick

- construction - thin

- object - thin

- construction - thick

- After all lines are drawn on a roof plan, what is added next?

- notes

- plan label

- a and b

- none of the above

- If the slope of each plane is equal and the supporting walls are equal in height, where will the ridge be located?

- toward the left supporting wall

- toward the right supporting wall

- halfway between the two supporting walls

- none of the above