Self-check Rubrics for Drafting the Electrical Plan

Architectural Drawing & Design II

Unit 1: Residential Electrical Systems

Electrical Requirements

| 1 | Duplex convenience outlets should be a maximum of 12 ft. apart. |  |
| --- | --- | --- |
| 2 | Duplex outlets should be no more than 6 ft. from a corner, and there should be a duplex outlet in any wall over 3 ft. long. |  |
| 3 | Consider furniture layout, if possible, so that duplex outlets do not become inaccessible behind large pieces. |  |
| 4 | Place a duplex outlet next to or behind a desk or table where light is needed, or near probable reading area. |  |
| 5 | Place a duplex outlet near a fireplace. |  |
| 6 | Place a duplex outlet in a hallway. |  |
| 7 | Duplex outlets should be placed close together in kitchens. Any outlet within 5 ft. of water should be protected by the GFI (ground-fault circuit-interrupter). |  |
| 8 | Each bath, sink, or make-up table should have a duplex outlet. |  |
| 9 | Each enclosed bath or laundry (utility) room should have an exhaust fan. Rooms with operable windows do not require fans, although they are desirable. |  |
| 10 | A light outlet is necessary outside all entries and exterior doors. |  |
| 11 | A waterproof duplex outlet should be placed at a convenient exterior location, such as a patio. |  |
| 12 | Ceiling lights are common in children’s bedrooms for easy illumination; master bedrooms may have a switched duplex outlet for a bedside lamp. |  |
| 13 | The kitchen may have a light over the sink, although in a very well-lit kitchen this may be an extra light. |  |
| 14 | Bathroom lights are generally placed above the vanity. In a large bath recessed lights may be placed in the shower or above the tub. |  |
| 15 | Place switches in a manner that provides easiest control of lights. |  |
| 16 | Locate lights in a large closet or in any alcove or pantry that requires light. |  |
| 17 | Light stairwells well. |  |
| 18 | Place lights and outlets in garages or shops in relationship to their use. |  |
| 19 | Place exterior lighting properly to illuminate walks, drives, patios, decks or other high use areas. |  |
| 20 | Line quality and proper use of symbols. |  |