Section and Details Vocabulary & Basic Information

Architectural Drawing & Design II

Unit 2: Sections and Details

- Annotation: a note added by way of a comment or explanation

- Architectural Details: the isolated and enlarged drawings that, together with a set of plans and elevations, form a complete set of working drawings; the most important drawings in the architectural set next to the plan

- Cornice Detail: shows all of the construction at the intersection of the walls and the roof, including the roof overhang

- Detail Drawing: a drawing, at a relatively large scale, of a part of a building with dimensions or other information for use in construction

- Footing: poured concrete base upon which the foundation wall rests

- Footing Detail: shows the footing and the foundation wall

- Section: a representation of an object as it would appear if cut by a plane, showing its internal structure

- Section Detailing: cross sections of the structure used to show the details of foundation, floor, ceiling and roof

- Sill Detail: shows the construction at the top of the foundation

- The details drawn by the architect or drafter serve as a basis for the shop drawings prepared by the contractor and subcontractors.

- Scales for Details: ½” to full size

- If the construction of a house is identical throughout, it can include only one wall section.

- A typical wall section includes three parts:

  - Footing Detail

  - Sill Detail

  - Cornice Detail

14. Footing Size: twice as wide and the same height as the thickness of the foundation

wall

15. Most common types of residential construction:

  - Frame Wall

  - Brick Veneer Wall

  - Brick Wall

  - Brick and Concrete Block Wall

  - Concrete Block Wall

16. Other Types of Detail Drawings:

- Window Details

- Door Details

- Fireplace Details

- Stair Details

- Structural Details (lintels, beams, girders, columns, and built-in installations

17. Elevations and Sections contain only vertical dimensions. No horizontal dimensions

should be placed on elevation drawings.