KEY: Unit Test: Sections and Details

Architectural Drawing & Design II

Unit 2: Sections and Details

A.  Matching (Match the following definitions to the correct word):

__F__  1. poured concrete base upon which the foundation		A. Sill Detail

wall rests

- Cornice Detail

__E__  2. a note added by way of a comment or explanation

- Footing Detail

__H__  3. a representation of an object as it would appear if

cut by a plane, showing its internal structure			D.  Detail Drawing

__A__  4. shows the construction at the top of the foundation		E.   Annotation

__I__   5. cross sections of the structure used to show the details	F.   Footing

of foundation, floor, ceiling and roof

G.  Architectural Details

__C__  6. shows the footing and foundation wall

H.  Section

__B__  7. shows all of the construction at the intersection of

the walls and the roof, including the roof overhang		I.   Section Detailing

__D__  8. a drawing, at a relatively large scale, of a part of a

building with dimensions or other information for

use in construction

__G__  9. the most important drawings in the architectural set

next to the plan

B.  Listing (List the information needed for the following):

- List 4 of the 5 most common types of residential construction:

  - ___Frame Wall___________________________________________

  - ___Brick Veneer Wall______________________________________

  - ___Brick Wall____________________________________________

  - ___Brick and Concrete Block Wall____________________________

Concrete Block Wall____________________________________

- List the 3 parts of a typical wall section:

  - ___Footing Detail________________________________

  - ___Sill Detail___________________________________

  - ___Cornice Detail________________________________

- List 4 other types of detail drawings that can be included in a set of plans:

  - ____Window Detail______________________________

  - ____Door Detail_________________________________

  - ____Fireplace Detail______________________________

  - ____Stair Detail__________________________________

____Structural Detail______________________________

C.  True or False (Tell whether the following statements are true or false):

___FALSE___ 1. Elevations and sections contain both vertical and horizontal dimensions.

___TRUE___   2. A footing size is twice as wide and the same height as the thickness of the 			             foundation wall.

___FALSE__   3. A set of house plans should always include at least two wall sections.

___TRUE_       4. Architectural details are the isolated and enlarged drawings that, together with 		             a set of plans and elevations, form a complete set of working drawings.

___TRUE____ 5. The details drawn by the architect or drafter serve as a basis for the shop 		                        drawings prepared by the contractor and subcontractors.

D.  Short Answer (Answer the following questions in complete sentences):

- Why are details needed with a set of drawings?

Answers will vary.

- Why do dimensions have to be included on all drawings?

Answers will vary.

- What special elements in a house would cause one to need extra sections and details?

Answers will vary.