Sill & Floor Construction Vocabulary

Architectural Drawing & Design II

Unit 3: Wall Sections

- Anchor Bolt: a threaded rod inserted in masonry construction to anchor the sill plate to the foundation

- Beam: a structural member transversely supporting a load

- Bridging: small wood or metal members that are inserted in a diagonal position between the floor joists at midspan to act as both tension and compression members for the purpose of stiffening and spreading concentrated loads

- Cantilever: a projecting beam or structural member anchored at only one end

- Crawl Space: the shallow space below the floor of a house built above the ground; usually surrounded with the foundation wall

- Curtain Wall: an exterior wall that provides no structural support

- Dead Load: the weight of the structure itself and the permanent components fastened to it

- Decking: material of concrete, asbestos, or steel, in the form of self-supporting flooring or roofing units laid between joists or rafters.

- Dimension Lumber: framing lumber that is 2 in. thick and from 4 to 12 inches wide

- Double Header: two or more timbers joined for strength

- Finished Floor: material attached over the subflooring

- Firecut: the angular cut at the end of a joist designed to rest on a brick wall

- Footing: poured concrete base upon which the foundation wall rests

- Foundation: the supporting portion of a structure below the first-floor construction or below grade including the footings

- Foundation Wall: a wall usually made of concrete poured directly on undisturbed soil to a minimum depth of about twelve inches

- Grade: the surface of the ground around a building

- Header: a framing member crossing and supporting the ends of joists, studs, or rafters so as to transfer their weight to parallel joists, studs, or rafters.

- Joist: a horizontal structural member supported by bearing walls, beams, or girders in floor or ceiling framing

- Keyway: a longitudinal groove in a footing providing a key for newly poured concrete.

- Laminated Beam: a beam made of superimposed layers of similar materials by joining them with glue and pressure

- Live Load: loads on a building such as wind, snow, and people

- Moisture Barrier: a sheet material that retards the passage of vapor or moisture into walls, floors, and ceilings

- Post-and-Beam Construction: a type of building frame in which roof and floor beams rest directly over wall posts

- Reinforced Concrete: concrete with steel bars or webbing embedded for strength

- Sill: the wood member placed directly onto the foundation wall in wood frame construction

- Span: the horizontal distance between supports for joists, beams, or trusses

- Subflooring: any material nailed directly to floor joists

- Termite Shield: sheet metal placed over masonry to prevent the passage of termites into wood

- Truss: structural member arranged and fastened in triangular units to form a rigid framework for support of loads over a long span

- Weep Holes: small holes in masonry cavity walls used to release water accumulation to the exterior