KEY: Sill and Floor Construction Test

Architectural Drawing and Design II

Unit 3: Wall Sections

Fill-in-the-Blank (Use one of the words from the word bank for each of the following):

Word Bank:

Anchor Bolt		Beam			Bridging		Cantilever

Crawl Space		Curtain Wall		Dead Load		Decking

Dimension Lumber	Double Header	Finished Floor		Firecut

Footing		Foundation		Foundation Wall	Grade

Header			Joist			Keyway		Laminated Beam

Live Load		Moisture Barrier	Reinforced Concrete

Sill			Span			Subflooring

Termite Shield		Truss			Weep Holes

Post-and-Beam Construction

- The weight of the structure itself and the permanent components fastened to it is called

a/an   ___DEAD LOAD___________________________.

- A___FOOTING____________________________ is a poured concrete base upon which

the foundation wall rests.

- The surface of the ground around a building is called the ____GRADE______________.

- The wood member placed directly onto the foundation wall in wood frame construction is

called the ___SILL_______________________.

- Small holes in masonry cavity walls used to release water accumulation to the exterior

are called ___WEEP HOLES_______________________.

- Loads on a building such as wind, snow, and people are called ___LIVE LOADS______.

- An exterior wall that provides no structural support is called a/an

___CURTAIN WALL________________________.

- A structural member transversely supporting a load is called a/an

_____BEAM______________________________.

- A type of building frame in which roof and floor beams rest directly over wall posts is

called _POST-AND-BEAM CONSTRUCTION______________________.

- A structural member arranged and fastened in triangular units to form a rigid framework

for support of loads over a long span is called a/an ____TRUSS____________________.

- The horizontal distance between supports for joists, beams, or trusses is called the

_____SPAN________________________.

- The angular cut at the end of a joist designed to rest on a brick is called a/an

_____FIRECUT_____________________.

- The shallow space below the floor of a house built above the ground is called the

_____CRAWL SPACE_________________.

- Small wood or metal members that are inserted in a diagonal position between the floor

joists at midspan to act as both tension and compression members for the purpose of

stiffening and spreading concentrated loads is called ___BRIDGING______________.

- The supporting portion of a structure below the first-floor construction or below grade

including the footings is called the ___FOUNDATION__________________.

- Any material nailed directly to floor joists is called __SUBFLOORING______________.

- A framing member crossing and supporting the ends of joists, studs, or rafters so as to

transfer their weight to parallel joists, studs, or rafters is called a/an

_____HEADER________________________.

- A ____FOUNDATION WALL________________________ is a wall usually made of

concrete poured directly on undisturbed soil to a minimum depth of about twelve inches.

- A horizontal structural member supported by bearing walls, beams, or girders in floor and

ceiling framing is called a/an ____JOIST______________________.

- A threaded rod inserted in masonry construction to anchor the sill plate to the foundation

is called a/an ____ANCHOR BOLT______________________.