Wall & Ceiling Construction Vocabulary

Architectural Drawing & Design II

Unit 3: Wall Sections

- Anchor: any fastener, usually metal, used to attach joists, trusses, posts, etc. to masonry or masonry materials

- Batt: a type of insulation designed to be installed between framing members

- Blocking: small wood pieces in wood framing used to anchor or support other major members

- Brick: a small masonry unit made from clay and baked in a kiln

- Brick Veneer: a facing of brick on the outer side of wood frame or masonry

- Building Paper: heavy paper used in construction of frame buildings to block drafts, for insulation

- Built-up Roof: a roofing composed of layers of asphalt felt laminated with coal tar, pitch, or asphalt; top finished with crushed slag or gravel

- Corner Brace: a diagonal brace at the corner of a wood frame wall to stiffen and prevent cracking

- Cornice: the molded projection of the roof overhang at the top of the wall

- Cross Bracing: boards nailed diagonally across studs or other boards to make framework rigid

- Drywall: interior wall covering, such as gypsum board or plywood, which is applied in large sheets or panels

- Eaves: the lower portion of the roof that overhangs the wall

- Expansion Joint: a flexible joint used to prevent cracking or breaking because of expansion and contraction due to temperature changes

- Face Brick: brick of better quality used on the face of a wall

- Fascia: a vertical board nailed onto the ends of the rafters

- Firestop: consists of 2 x 4 cross blocking between studs

- Flashing: sheet metal or other material used in roof and wall construction to protect a building from water seepage

- Furring: thin strips of wood fastened to walls or ceilings to provide a space for insulation, to help prevent moisture transmission, and to provide a level surface for paneling or other surface finished treatment

- Gutter: a metal or wood trough for carrying water from a roof

- Gypsum Board: gypsum sheets covered with paper, which are fastened to walls and ceilings with nails or screws

- Insulation: materials for obstructing the passage of sound, heat, or cold from one surface to another

- Joist: a horizontal structural member that supports the floor or ceiling system

- Metal Wall Tie: corrugated metal strips used to tie masonry veneer to wood walls

- Pilaster: a rectangular pier attached to a wall for the purpose of strengthening the wall

- Rafter: a roof structural member running from the wall plate to the ridge

- Ridge: the top edge of a roof where two slopes meet

- Ridge Board: the board placed on edge at the ridge of the roof into which the upper ends of the rafters are fastened

- Roof Sheathing: the board or sheet material fastened to the roof rafters on which the shingles or other roof material is placed

- Sheathing: the structural covering, usually wood boards or plywood, used over studs or rafters of a structure

- Shingles: thin pieces of material laid in overlapping rows to cover the roofs of buildings

- Siding: the outside finish covering on a frame wall

- Soffit: the underside of an overhang such as a cornice

- Sole Plate: the horizontal member of a frame wall resting on the rough floor to which the studs are nailed

- Stud: the vertical framing member of a wall

- Top Plate: wood members that form the top of a framed wall