Fireplace Vocabulary

Architectural Drawing & Design II

Unit 4: Fireplace Details

- Ash Dump: an opening in the floor of a fireplace through which ashes are swept to an ash pit

- Ash Pit: an enclosed opening below a fireplace to collect ashes

- Damper: a movable plate that regulates the draft through a flue or duct

- Fireclay: a refractory mortar used to lay firebrick in the bed and walls of a fireplace

- Flue: the passage in a chimney through which smoke, gases, and fumes escape to the outer air

- Hearth: the incombustible floor in front of and within the fireplace

- Lintel: an angle iron which must be placed across the span of the fireplace opening; used to support the masonry units that are laid over the opening

- Saddle: a small gable roof placed in back of a chimney on a sloping roof to shed water and debris

- Single-face: a fireplace which has only a single opening

- Smoke Chamber: the enlarged portion of a chimney flue directly above the fireplace

- Smoke Shelf: deflects cold air flowing down the chimney into the rising warm air

- Three-face: a fireplace which is open on three sides; also known as three-way fireplace

- Throat: offsets the draft above the chamber

- Two-face (adjacent): a fireplace which is open on the front and one side

- Two-face (opposite): a fireplace which is open on both the front and back sides