Basic Cabinet Information

Architectural Drawing & Design II

Unit 5: Millwork Details

- Standard Base Cabinets: 34 ½”  high, 24” deep, width increments in 3” multiples (15”, 18”, 21”, etc.)

- Wall Cabinets: 12” – 13” deep, 12” – 30” high (3” increments), 12” – 36” wide (3” increments)

- Wall Cabinets: Run to the top of standard 8’-high wall; use top 14” for dead storage.

- Wall and base cabinets should be identified in the plan view. The wall cabinets are shown with hidden lines while the base units are shown as object lines.

- Provide maximum kitchen storage with both base cabinets and wall cabinets where possible.

- The location of kitchen cabinets, bathroom vanities, fixtures, and appliances must be indicated on the floor plan. Use standard symbols which represent the correct sizes.

- Allow at least 40 square feet of shelf space for general kitchen storage; add an additional 6 square feet for each person living in home.

- U-shaped and two-wall type kitchens: Allow between 4’ – 6’ of floor space between faces of opposite cabinets.

- Make modifications of the basic arrangement without destroying the function of the work center.