Site Plan Vocabulary Test

Architectural Drawing & Design II

Unit 6: Site Plans

Matching (Match the following words with the correct definitions):

_____ 1.  a plan view drawing which shows the site, location,		A.  Contour Lines

and orientation of the buildings on the property

B.   Landscape Plan

_____ 2.  contour lines that are not the result of a survey,

represented by short dashed lines					C.  Site Analysis

_____  3.  the vertical distance between two adjacent contours		D.  Site Plan

_____ 4.  refers to site characteristics such as contour of the 		E.  Bearing Line

land, trees, or other natural features

F.  Property Lines

_____ 5.  a plan or map, as of land

G.  Zoning Ordinances

_____ 6.  a horizontal direction expressed in degrees east or west

of a true or magnetic north or south direction			H.  Estimated Contours

_____ 7.  a complete analysis to determine the ideal location and	 I.   Survey

placement of the structure on the site

J.  Contour Interval

_____ 8.  building restrictions which regulate size, location, and

type of structures to be built in specific areas			K.  Plat

_____ 9.  arrow on a site plan that shows which direction is north	L.  Topography

_____10. lines that indicate the required distances of the location	M.  Meridian Arrow

of a structure in relation to the boundaries of the property

N.  Setback Lines

_____11. an area of a piece of property giving rights to another

for the purpose of placing power lines and drains		O.  Easement

_____12. the length and bearing of each line of the site boundary

_____13. a line connecting points which have the same elevation

on a site plan

_____14. a description of the measure and marking of the land, including

maps and field notes, which describe the property

_____15. a plot plan that is designed to show the type and

placement of trees, shrubs, and pools on the site