Some Hints on Mechanical Drawing or Drawing by Hand:

1)  Use edge of triangles or other straight edge to draw straight lines; never use a scale.

2)  Make construction lines thin and light.  Make final lines dark and sharp.

3)  Make very light construction lines first. After the drawing has been completed with these, go back and darken the final lines.

4)  Visible Lines should all be the same thickness and darkness, even those made with a compass.

5)  Compass should lean at an inclined angle of about 30 degrees from vertical into the direction being drawn.

6)  Keep your compass lead sharpened.

7)  Try not to slide triangles or scales on your drawing surface.  Always lift and place if possible.

base

slider

8)  Use two triangles to create

parallel lines.  Hint: think about

placing the triangles’ hypotenuses

together.

9)  When lettering by hand, print using simple stroke uppercase lettering maintaining consistent height and width.  Guidelines are helpful. Suggested height of lettering is 1/8".

10)  Graphite is both your enemy and your friend.  Getting it on everything makes your drawing sloppy and dirty.