Centering three-view drawings – prep notes

There are various ways to center three views on a sheet, but this one uses a standard distance between the views as an example. This example uses 1” between the views.

Have students

- lay out border lines and title block with construction lines.

- run the diagonals to find the center of the drawing space.

- determine the width, height, and depth of the object.

- add the width + 1” + depth for overall width of the box.

- add the height + 1” + depth for overall height of the box.

- divide the overall width by 2. Hold the scale horizontally and place that halved sum on the center point and mark 0 on the left of the scale and the overall width measurement on the right.

- divide the overall height by 2. Hold the scale vertically and place that halved sum on the center point and mark 0 on the left of the scale and the overall width measurement on the right.

- draw the box using construction lines.

- mark off the width, 1”, and depth measurements horizontally left to right.

- mark off the height, 1”, and depth measurements vertically top to bottom.

- construct the boxes for the views with construction lines.

- run a 45 degree construction (miter) line from upper right corner of front view to corner of total box to check accuracy and for projection.