Multiview Projection - key

Name ____________________

- What is the purpose of a multiview drawing?

- How many potential views are there of an object?  6

- List the three principal dimensions of an object.  Width, depth, height

- In a drawing that shows the top, front, and right side views, which two views show depth?

Right side and top

- Which view shows depth as a horizontal line?  Side

- The height, width, and depth of a non-symmetrical object can be shown with a minimum of how many orthographic projection views?  3

- What dimension does the top and front view have in common?   Width, height, or depth?

- Which view shows depth as a vertical line?  Top

- What dimension does the side view and front view have in common?   Width, height, or depth?

- Generally, what factors determine which view is placed as the front view? Has fewest hidden lines and most contours

- What dimension does the top and right side view have in common?   Width, height, or depth?

- What would be a reason you might choose a left side view as opposed to a right side?

Left is more descriptive, fewer hidden lines, etc.

- The angle on an inclined surface will show as an angle how many times in a three-view drawing?  Once

- Height is shown in which two views? Front and side

- A sphere can be described in how many views? One

- A symmetrical object is usually described by how many views? Two

- What is the definition of symmetrical? Identical on both sides of an axis or center line

- Width is shown in which two views? Front and top

- Depth is shown in which two views? Top and side