## Network Relationships to the OSI Model

| OSI Model Layer | OSI Model Name | Pneumonic | # Equipment | Equipment Purpose | # Data | # Protocols | Words to Remember | TCP/IP Model |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Layer 7 |  | A |  |  | D |  | Browsers | Application |
| Layer 6 |  | P |  |  |  |  |  | Application |
| Layer 5 |  | S |  |  |  |  |  | Application |
| Layer 4 |  | T |  |  | S |  |  | Transport |
| Layer 3 |  | N |  |  | P |  |  | Internet |
| Layer 2 |  | D |  |  | F |  |  | Network Access |
| Layer 1 |  | P |  |  | B |  |  | Network Access |

## Network Relationships to the OSI Model Answers

| OSI Model Layer | OSI Model Name | Pneumonic | # Equipment | Equipment Purpose | # Data | # Protocols | Words to Remember | TCP/IP Model |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Layer 7 | Application | All | Computer | Regular Computer or A Special Gateway. Used to combine networks using different communication protocols | Data | Redirector, FTP, Telnet, SMTP, SNMP, Netware Core, | Browsers | Application |
| Layer 6 | Presentation | People |  |  |  |  | Common Data Format | Application |
| Layer 5 | Session | Seem |  |  |  | NFS, SQL, RPC, X-Win | Dialogues and Conversations | Application |
| Layer 4 | Transport | To | Computer |  | Segment | TCP and UDP | Quality of Service, and Reliability | Transport |
| Layer 3 | Network | Need | Router | Segment Network into Smaller Broadcast Domains | Packet | Routable Protocols. (IP, IPX, AppleTalk) | Path Selection, Routing, and Addressing | Internet |
| Layer 2 | Data Link - LLC - MAC | Data | Bridge (2 Ports) or Switch (Multi-port) and NIC | Segment Network into Smaller Collision Domains | Frame | NDIS, ODI, MAC Address, Ether Talk | Frames and Media Access Control (MAC) | Network Access |
| Layer 1 | Physical | Processing | Repeater (2 Ports), Hub (Multi-port), Cabling | One Collision AND One Broadcast Domain | Bit | Physical | Signals and Media | Network Access |