Business and Computer Science

PATHWAY:  		Computer Networking

COURSE:  		Networking Systems

UNIT 14: 		Routing

Annotation:

In this unit students will explore LAN, MAN and WAN trends and issues.This unit includes references to the Cisco Networking Academy Discovery curriculum and Instructor Interactive course guides. http://cisco.netacad.net

Grade(s):

|  | 9th |
| --- | --- |
|  | 10th |
| X | 11th |
| X | 12th |

Time:

10 Hours

Prepared by:

Douglas L. Arnold / Cisco Networking Academy

Students with Disabilities:

For students with disabilities, the instructor should refer to the student's IEP to be sure that the accommodations specified are being provided. Instructors should also familiarize themselves with the provisions of Behavior Intervention Plans that may be part of a student's IEP. Frequent consultation with a student's special education instructor will be beneficial in providing appropriate differentiation.

GPS Focus Standards:

BCS-NTS-1. Students will explore local-area network (LAN), metropolitan area network

(MAN), and wide-area (WAN) trends and issues including the basics of

telecommunications and use in the interconnection of networks.

a. Explain the advantages and disadvantages of a network system.

b. Identify the three major network classifications: LAN, MAN, and WAN.

c. Identify the basic network topologies.

d. Compare and contrast a peer-to-peer network with a client/server network.

e. Describe how data is packaged and transmitted.

f. Explain the purpose of a protocol.

g. List the common networking protocols.

h. Explain the purpose of general network devices such as a hub, repeater, switch, and

gateway.

i. Identify the major standards organizations.

j. Identify and explain the purpose of the IEEE 802 standards.

k. List and explain the purpose of each OSI layer.

GPS Academic Standards:

Academic Standards:

MM2P4 Students will make connections among mathematical ideas and to other disciplines.

ELA10RL5 The student understands and acquires new vocabulary and uses it correctly in

reading and writing.

ELA10RC3 The student acquires new vocabulary in each content area and uses it correctly.

ISTE/NETS:

ITEA - 5. Students will develop an understanding of the effects of technology on the environment.

ITEA - 8. Students will develop an understanding of the attributes of design.

ITEA - 12. Students will develop the abilities to use and maintain technological products and systems.

ITEA - 17. Students will develop an understanding of and be able to select and use information and communication technologies.

Enduring Understandings:

Students will identify the fundamental principles and the installation, configuration, optimization, and upgrade of local area networks, client server relationships and network services. Students will also identify the tools, diagnostic procedures, and troubleshooting techniques for maintaining computer networks components, as well as for performing preventive maintenance.

Essential Questions:

    - Can you identify routers and routing tables as the method by which network devices direct messages across network?

    - Can you identify static routing as a method for routing small networks?

    - Can you identify routing protocols as a tool for automated routing updates?

    - Can you identify routing Information Protocol (RIP) as a pervasive internal routing protocol?

    - Can you determine the appropriate routing protocol for a network?

    - Can you identify the procedures used when configuring a router with RIP?

    - Can you identify autonomous systems (AS) as a set of networks controlled by a single administrative authority?

    - Can you identify Exterior Gateway Protocols (EGP) that enable routing between autonomous systems?

    - Can you identify the process for routing packets across the Internet?

    - Can you identify the role of the Internet Service Provider (ISP) in support of routing across the Internet?

    - Can you configure and verify Border Gateway Protocol (BGP)?

Knowledge and Skill from this Unit:

    - Students will be able to define the router as the network device responsible for routing packets across interior and exterior networks.

    - Students will be able to explain the role of routing tables to enable network routing.

    - Students will be able to differentiate static routing from dynamic routing.

    - Students will be able to demonstrate how to use the router IOS to read a routing table on a Cisco router.

    - Students will be able to differentiate the four types of routes - directly-connected, static, dynamically-updated, and default routes.

    - Students will be able to demonstrate the process for configuring a Cisco router.

    - Students will be able to differentiate RIP from RIP2.

    - Students will be able to differentiate the interior routing protocols from exterior routing protocols.

    - Students will be able to demonstrate the process for configuring RIP on a Cisco router.

    - Students will be able to differentiate “distance vector routing” from “link state routing”.

    - Students will be able to explain the criteria for choosing the proper routing protocol.

    - Students will be able to define an Autonomous System and differentiate an AS from a medium and small organization network.

    - Students will be able to explain and illustrate the AS numbering system.

    - Students will be able to differentiate interior routing protocols from external routing protocols.

    - Students will be able to illustrate the concepts of border gateways and exterior routers.

    - Students will be able to explain Border Gateway Protocol (BGP).

    - Students will be able to illustrate the flow of packets as they move through AS’s and are routed from origin to destination.

    - Students will be able to explain how ISPs use exterior protocols to monitor and manage Internet traffic.

    - Students will be able to demonstrate and lead a configuration exercise implementing BGP on a Cisco router.

Assessment Method Type:

|  | Pre-test |
| --- | --- |
| X | Objective assessment - multiple-choice, true- false, etc. |
|  | __ Quizzes/Tests _X_ Unit test |
| X | Group project |
| X | Individual project |
| X | Self-assessment - May include practice quizzes, games, simulations, checklists, etc. |
|  | __ Self-check rubrics __ Self-check during writing/planning process __ Journal reflections on concepts, personal experiences and impact on one’s life __ Reflect on evaluations of work from teachers, business partners, and competition judges __ Academic prompts __ Practice quizzes/tests |
| X | Subjective assessment/Informal observations |
|  | __ Essay tests _X_ Observe students working with partners _X_ Observe students role playing |
|  | Peer-assessment |
|  | __ Peer editing & commentary of products/projects/presentations using rubrics __ Peer editing and/or critiquing |
| X | Dialogue and Discussion |
|  | __ Student/teacher conferences _X_ Partner and small group discussions _X_ Whole group discussions __ Interaction with/feedback from community members/speakers and business partners |
| X | Constructed Responses |
|  | __ Chart good reading/writing/listening/speaking habits _X_ Application of skills to real-life situations/scenarios |
|  | Post-test |

Assessment Attachments and / or Directions:

Unit 4 Assessment:

Instructional planning:

	LESSON 1:  ENABLING ROUTING PROTOCOLS

1.	Identify the standards. Standards should be posted in the classroom.

2.	Review Essential Question(s). Post Essential Questions in the classroom.

3.	Identify and review the unit Anticipation guide / study guide or vocabulary.

4.  Teaching Goals: Your Challenge as a Teacher in This Section Is To:

    - Introduce the router as the network device responsible for routing packets across interior and exterior networks.

    - Explain the role of routing tables to enable network routing.

    - Differentiate static routing from dynamic routing.

    - Demonstrate how to use the router IOS to read a routing table on a Cisco router.

    - Differentiate the four types of routes - directly-connected, static, dynamically-updated, and default routes.

    - Demonstrate the process for configuring a Cisco router.

    - Differentiate RIP from RIP2.

    - Introduce and differentiate the interior routing protocols from exterior routing protocols.

    - Demonstrate the process for configuring RIP on a Cisco router.

    - Differentiate “distance vector routing” from “link state routing”.

    - Explain the criteria for choosing the proper routing protocol.

5.	Activities:

Reference interactive course guide (ICG):

- Review How to Teach: 6.1 ICG

- Review Critical Concepts 01: 6.1 ICG

- Review Critical Concepts 02: Misconceptions and Errors 6.1 ICG

- Review Discussion 1-20 ICG 6.1

- Reflection 3.1: Assignments

Reflections Activity:

1.	How Many Ways:

a.	Provide students with a set of roadmaps.

i.	Maps can be downloaded from the Internet and can be obtained, in many countries, free from the 				AAA, CAA, RAC, and other organizations that provide trip planning and roadside assistance.

b.	Provide students with a list of local or national starting points and destinations, for example, from London 		to Edinburgh. Make sure that the local/national origins and destinations have at least two different ways 			that can be “mapped”.

c.	Have the students prepare different “routes” from the origin to destination, documenting the 				intersections, route numbers, etc.

d.	Have the students share the results of their investigations, comparing the routes along the way.

e.	Now extend the activity by imaging that there was a bottleneck at one of their major intersections. How 			would they, on the fly, reroute their trip to avoid this problem?

f.	Think back to the module, what would be an example of a statically assigned route, and what would be an 		example of a dynamic route when driving from origin to destination?

g.	It may be interesting, if time permits, to have the students also use web-based mapping tools to compare 			their route with the “shortest” route recommended by web mapping tools.

2.	ANDING Activities:

a.	Provide students with 10 dotted decimal IP and subnet addresses.

b.	Have them perform ANDING to determine the network and subnet portion of the destination IP address.

3.	Router Connectivity: Think back to the OSI and TCP/IP layer models we discussed in the first course. Each of the 	following commands can be used to test router connectivity; your job is to research at what layer of the OSI 	model each of these commands works. Which layer would you start troubleshooting connectivity? Why?

Click on “Activity 10” in the “Activities” button above and display the chart.

4.	Static Routing:

a.	Review the syntax for the command ip route – destination_network subnet_mask gateway_address

i.	ip route command for next-hop IP address

ii.	RouterX(config)#ip route 198.10.1.0 255.255.255.0 200.1.1.2 (next-hop IP address)

b.	Provide students with ten examples following the following schema:

i.	Router (number) (a minimum of two)

ii.	IP address of router

iii.	Destination network

iv.	Subnet mask

v.	Gateway address

c.	Students should configure the static route and write the IOS command they will use for each of the ten 			examples.

d.	Discuss the results.

5.	Static Routing Lab:

Estimated Time: 60 min.

Objectives:

a.	Subnet a Class C IP address and apply to the standard lab topology

b.	Configure static routes for the WAN

Background: In this lab, you will configure static routing for the wide area network without using a routing 	protocol such as RIP or IGRP. Static routes are routes that are manually configured by the network administrator. 	Routing updates are not sent on a link if it is defined by only static routes, thus conserving bandwidth and router 	CPU resources. Static routes are typically more secure than dynamic routes.

Click on “Activity 11” in the “Activities” button above to access the charts and diagrams that are to be displayed 	and the directions for the lab.

6.	Algorithms in Our Lives: Tell students, “It is estimated that over 100,000 lives in the United States alone have 	been saved with the use of algorithms in air bags and anti-lock breaking systems.” Ask students to:

a.	Research the use of algorithms in cars, video games, and computers, even to solve the Rubik’s Cube.

b.	Their research should include what the algorithms are used for, i.e. “do” within the process. Model this 			or students with the following example:

i.	In 4 wheel ABS systems a speed sensor measures the speed of all four wheels

ii.	Then the microprocessor “processes” the information, using an algorithm which identifies abnormal 				deceleration

iii.	Then another algorithm is applied which determines how hard to “pump” the brakes (opening and 				losing valves) to keep the wheels from locking up until all the wheels are at or near the same speed.

c.	You may choose to give students specific algorithms to research, i.e. anti-lock brakes, encryption, file 			compression, video games, high definition television, algorithms used in computer animation, training 			simulations for military and safety (firefighters and police officers), or just allow them to search for 			algorithms.

d.	Students should prepare a one-paragraph outline on the use of algorithms in the area/field they 				researched to present to the class.

i.	Remind students to present the application (use of) the algorithm, for example, anti-lock brakes.

ii.	The purpose of the algorithm (what it does), for example, identifies abnormal deceleration

iii.	How the algorithm is applied (used), for example, determines how hard to pump the brakes

iv.	Any additional uses/future developments for the use of algorithms in this device

7.	Disadvantages of Distance-Vector Routing: Tell students, “We have discussed the advantages of using distance-	vector routing over static routing. There are, however, also some disadvantages to the use of distance-vector 	routing of which you should be aware. Using the Internet and classroom resources, research the following 	disadvantages:

a.	Convergence

b.	Routing loops including Count to Infinity

c.	Scale

Prepare a ¾ to 1 page summary report of your research. Be prepared to discuss the results of your research with 	the class.

8.	Selecting a Routing Protocol: Videogames for All, an international used videogame company, wants to 	implement wide area network links between 100 stores spread across 20 countries. Videogames for All has 	several requests:

a.	Convergence needs to be as fast as possible

b.	Because of slow wide area links in some developing countries routing table updates need to be kept to a 			minimum

A member of the Videogames for All tech support team feels that a distance-vector routing protocol will be the 	best choice. Which protocol do you suggest? Why? Prepare a short (less than one page) report of your suggested 	protocol; explaining why the protocol you selected is the best choice for this company, demonstrating the 	advantages/disadvantages of this protocol over others that might have been selected.

9.	Configuring RIP: Wanda and Sienna are configuring RIP on a client’s router, but need some advice. The network 	number is 202.208.4.0. There are three interfaces on the router:

a.	202.208.4.25

b.	202.208.4.92

c.	202.208.4.112

The subnet mask is 255.255.255.241. The host name for the router is "Jacksonville". What mode will Wanda and 	Sienna be in to use the router rip command? What will the prompt look like? What will the prompt read after 	the router rip command is entered? What will be the next commands that they answer?

References

1.	Howard, Bill. (May 23, 2006). Show Report: Inrix Real Time Traffic. Retrieved April 3, 2007 from 	http://www.technoride.com/article/Show+Report+Inrix+RealTime+Traffic/179071_1.aspx.

2.	Dennis, Tony (January 5, 2006). Real Time Traffic Launched for Symbian.  Retrieved April 3, 2007 from 	http://www.theinquirer.net/default.aspx?article=28730

3.	Wikipedia. (April 16, 2997). E-mail Spam. Retrieved April 5, 2007 from http://en.wikipedia.org/wiki/E-	mail_spam.

4.	Dennison University (October 19, 2005).  Algorithms are Everywhere. Retrieved April 5, 2007 from 	http://personal.denison.edu/~havill/algorithmics/algs.html.

5.	Wikitionary. (April 13, 2007). Algorithms. Retrieved April 6, 2007 from http://en.wiktionary.org/wiki/algorithm.

6.	Nice, Karim. (2007). How Anti-Lock Brakes Work. Retrieved April 6, 2007 from 	http://auto.howstuffworks.com/anti-lock-brake.htm.

7.	Parkhurst, William. (2004). Routing First Step. San Jose:  Cisco Press

8.	For Activity 05: Working from left to right: 1 and 2 are from Dennison University (October 19, 2005).  Algorithms 	are Everywhere. Retrieved April 5, 2007 from http://personal.denison.edu/~havill/algorithmics/algs.html. 3 is 	from Answers.com (2007). Greedy Algorithms. Retrieved April 5, 2007 from  	http://content.answers.com/main/content/wp/en/3/34/Greedy_algorithm_change_diagram.jpg

9.	For Activity 07: Hurricane Watch (September 5, 2005). Retrieved April 5, 2007 from 	http://outhouserag.typepad.com/hurricane_watch/pictures/index.html

	LESSON 2: EXTERIOR ROUTING PROTOCOLS

1.	Identify the standards. Standards should be posted in the classroom.

2.     Review Essential Question(s). Post Essential Questions in the classroom.

3.	Identify and review the unit Anticipation guide / study guide or vocabulary.

4.     Teaching Goals: Your Challenge as a Teacher in This Section Is To:

    - Define an Autonomous System and differentiate an AS from a medium and small organization network.

    - Explain and illustrate the AS numbering system.

    - Differentiate interior routing protocols from external routing protocols.

    - Introduce and illustrate the concepts of border gateways and exterior routers.

    - Explain Border Gateway Protocol (BGP).

    - Illustrate the flow of packets as they move through AS’s and are routed from origin to destination.

    - Explain how ISPs use exterior protocols to monitor and manage Internet traffic.

    - Demonstrate and lead a configuration exercise implementing BGP on a Cisco router.

3.  	Activities:

Reference interactive course guide (ICG):

- Review How to Teach: 6.2 ICG

- Review Critical Concepts 01: 6.2 ICG

- Review Critical Concepts 02: Misconceptions and Errors 6.2 ICG

- Review Discussion 1-5 ICG 6.2

- Reflection 6.2: Assignments

Reflections Activity:

1.	ASN Investigations: Ask students, “How many ASN numbers are there? If I were to tell you ASN numbers 	are 2 to the 16th power, would that give you a hint? It is predicted that at the current rate of assignment 	(with an attempt to reclaim those ASN’s no longer used by an ISP or organization) that ASN’s will last, at 	the longest, until 2016.” ⁵ ⁶

Ask students to research the following:

a.	What will happen when we run out of ASN’s? What plans do APNIC and IANA have for expanding the 			numbering system for ASN’s?

i.	Are there problems in switching to a new numbering system?

b.	When you look at the ASN list, how many and what kinds of companies or organizations own multiple 		ASN’s? Why might these companies or organizations have a need for multiple ASN’s?

i.	Students should identify a minimum of four companies or organizations with multiple ASN’s.

c.	In looking at the list, do you see the name of companies or organizations that no longer exist?

i.	Students should identify a minimum of two companies or organizations with multiple ASN’s.

d.	Find the names of at least three companies or organizations that “surprise” you by having their own 			AS. For example, Harpo Productions, the company run by Oprah Winfrey, is number 26884.

e.	When you look at the ASN list, can you identify any companies that might have obtained an ASN due 			to issues of reachability?

i.	Remember – these are likely to be multinational companies who require multiple AS’s in order to 			do business.

To make this investigation more interesting you could split the class into “teams” and give teams points for each company located in 2 - 5, awarding a prize to the team with the highest point value.

Resources

Direct students to the following article on the past, present and future of ASN: http://www.cisco.com/web/about/ac123/ac147/archived_issues/ipj_9-1/autonomous_system_numbers.html

If this article becomes unavailable, students should search on Autonomous System Numbers, as ASN is an abbreviation used by many different groups.

Direct students to the following URL for a list of ASN numbers and affiliated names of organizations/businesses. http://bgp.potaroo.net/cidr/autnums.html

This article takes a substantial time to download; it is suggested that the instructor download and distribute the article electronically to the class.

References

1.	Internet Society.  (November 9, 2006). A Brief History of the Internet and Related Networks. Retrieved 	April 10, 2007 from http://www.isoc.org/internet/history/cerf.shtml.

2.	Internet Mapping Project. (June 6, 2006). Selected Datasets from the Internet Mapping Project. Retrieved 	April 10, 2007 from http://www.cheswick.com/ches/map/dbs/index.html.

3.	Huffaker, Brad. (March 30, 2006). Visualizing Internet Topology at a Macroscopic Scale. Retrieved April 8, 	2007 from http://www.caida.org/analysis/topology/as_core_network/.

4.	Huffaker, Brad. (March 2, 2006). Visualizing Internet Topology at a Macroscopic Scale. Retrieved April 8, 	2007 from http://www.caida.org/analysis/topology/as_core_network/ipv6.xml.

5.	Huston, Geoff. (January 2007) 32-bit AS Numbers – The View from the old BGP World. Retrieved April 7, 	2007 from http://ispcolumn.isoc.org/2007-01/asn32.html.

6.	Wikipedia. (April 11, 2007) Autonomous system (Internet). Retrieved April 7, 2007 from 	http://en.wikipedia.org/wiki/Autonomous_system_(Internet).

Summative Assessment: S2 Chapter 6 Quiz 6.0

ATTACHMENTS FOR LESSON PLANS

| Lesson 1-2: | - Unit Anticipation Guide - Unit Online Study Guide - Unit Online Study Guide Key - Unit Study Guide - Jeopardy ppt |
| --- | --- |

Web Resources:

- http://www.gns3.net/ - Open source Router / Network Simulator

- http://www.howstuffworks.com/computers

- See links in Reflection activities.

Materials & Equipment:

Cisco Networking Academy Interactive Course Guides and Discovery Curriculum.

21st Century Technology Used:

| X | Slide Show Software |  | Graphing Software |  | Audio File(s) |
| --- | --- | --- | --- | --- | --- |
| X | Interactive Whiteboard |  | Calculator |  | Graphic Organizer |
|  | Student Response System |  | Desktop Publishing | X | Image File(s) |
|  | Web Design Software |  | Blog |  | Video |
|  | Animation Software |  | Wiki | x | Electronic Game or Puzzle Maker |
|  | Email | X | Website |  |  |