CCNA Discovery					Module 5 Network Addressing

| 5.1 | IP Addresses and Subnet Masks |  |
| --- | --- | --- |
| 5.1.1 | Purpose of the IP Address |  |
| 1. | What is an IP address? | a logical network address that identifies a particular host |
|  | Packet Tracer Activity |  |
| 5.1.2 | IP Address Structure |  |
| 2. | How many bits are in an IP address | 32 |
| 3. | What is an octet? | four 8-bit bytes |
| 4. | What is dotted decimal notation? | The IP address presented in decimal value, separated by a decimal point |
| 5. | What is the most common form of IP address on the Internet? | IP version 4 (IPv4) |
| 6. | How many possible IP addresses are there using a 32-bit addressing scheme? | Over 4 billion |
| 7. | Write the binary number line? | 128, 64, 32, 16, 8, 4, 2, 1 |
|  | Binary Number Game |  |
| 5.1.3 | Parts of an IP Address |  |
| 8. | What two parts make up the IP address? | The network and the host |
| 9. | What is hierarchical addressing? | The network portion indicates the network on which each unique host is located |
|  | Activity |  |
| 5.1.4 | How IP Addresses and Subnet Masks Interact |  |
| 10. | What does the subnet mask signify? | Which part of the IP address is network and which part is host |
| 11. | What do the 1s in the subnet mask represent? | The network portion |
| 12. | How do you calculate the number of hosts that can be on a network? | Take the number 2 to the power of the number of hosts bits and subtract 2 |
| 13. | What is the maximum number of hosts available if you have 8 host bits? | 254 |
|  | Lab Activity – Number Conversions |  |
| 5.2.1 | IP Address Classes and Default Subnet Masks |  |
| 14. | How many classes of IP addresses are there? | 5 |
| 15. | What are the classes of IP addresses? | A, B, C, D, E |
| 16. | Which class of IP address is reserved for multicast use? | D |

| 17. | Write the address class and the range of the first octet range. | A 1-127 B 128 – 191 C 192 – 223 D 224 - 239 E 240 - 255 |
| --- | --- | --- |
| 18. | In a Class A address, how many octets represent the host portion? | one |
|  | Activity |  |
| 5.2.2 | Public and Private IP Addresses |  |
| 19. | What is RFC 1918? | A standard that reserves several ranges of addresses within each of the classes A, B, and C |
| 20. | What is the private network address for a class A network? | 10.0.0.0 |
| 21. | What are the private network addresses for a Class B network? | 172.16.0.0 – 172.31.0.0 |
| 22. | What are the private network addresses for a Class C network? | 192.168.0.0 – 192.168.255.0 |
| 23. | Why do companies use private addresses? | It provides a measure of security since they are only visible on the local network |
| 24. | What is a loopback address? | A private address used for the diagnostic testing of devices |
| 25. | What is the Class A loopback address? | 127.0.0.0 |
|  | Activity |  |
| 5.2.3 | Unicast, Broadcast and Multicast Addresses |  |
| 26. | What is the most common address on an IP network? | Unicast |
| 27. | A packet with a unicast destination address is intended for what? | A specific host |
| 28. | In a broadcast address, the packet contains what in the host portion of the destination IP address? | All ones in the host portion |
| 29. | Who receives a broadcast? | All hosts on a local network |
| 30. | What is the broadcast MAC address on Ethernet networks? | FF-FF-FF-FF-FF-FF |
| 31. | What is a multicast? | A source device sends a packet to a group of devices |
| 32. | What is the range of multicast addresses? | 224.0.0.0 to 239.255.255.255 |
| 33. | Give an example of where multicast addresses would be used? | Remote gaming |
| 34. | What does the multicast MAC address begin with? | 01-00-5E |
|  | Activity |  |
| 5.3.1 | Static and Dynamic Address Assignment |  |
| 35. | What is a static address? | One the network administrator has manually assigned |
| 36. | What are some advantages of the static addresses? | They are useful for networking devices that need to be accessible to clients on the network; increased control of network resources |
| 37. | What is DHCP? | Dynamic Host Configuration Protocol |
| 38. | What does DHCP do? | Automatically assigns IP address, subnet mask, default gateway, and other configuration information |
| 39. | What happens to the DHCP address once the host is powered down or taken off the network? | The address is returned to the pool for reuse |
| 5.3.2 | DHCP Servers |  |
| 40. | What does a DHCP server do? | Manages and assigns the IP addresses and ensures that all IP addresses are unique. |
| 5.3.3 | Configuring DHCP |  |
| 41. | How does a DHCP client get an IP address? | It sends a DHCP Discover message which is a broadcast with a destination IP address of 255.255.255.255 and a destination MAC address of FF-FF-FF-FF-FF-FF |
| 42. | What is the default IP for the Linksys wireless router? | 192.168.1.1 |
| 43. | What is the default lease time for DHCP addresses? | 24 hours |
| 5.3.3.3 | Packet Tracer Activity |  |
| 5.4.1 | Network Boundaries and Address Space |  |
| 44. | What does a router provide? | A gateway through which hosts on one network can communicate with hosts on different networks |
| 45. | What does the IP address assigned to the interface identify? | Which local network is connected directly to it |
| 46. | What is the default gateway? | The IP address of the router interface connected to the network where the host is attached |
| 5.4.2 | Address Assignment |  |

| 47. | What is the difference between an internal and an external network? | An external network is the network assigned to the Internet side of the integrated router |
| --- | --- | --- |
| 48. | What is the boundary between the local internal network and the external Internet? | The integrated router |
| 49. | How can hosts be connected to an ISP and the Internet? | Direct Connection; Connection Through an Integrated Router, Connection Through a Gateway Device |
| 5.4.3 | Network Address Translation |  |
| 50. | What is the function of an integrated router? | It receives a public address from the ISP and provides private addresses to local network clients |
| 51. | What is Network Address Translation (NAT)? | A private (local) source IP address is translated to a public (global) address |
| 5.4.3.2 | Packet Tracer Activity |  |