## Module 26107-08
Hand Bending

Lesson 1 of 3: Introduction to Hand Bending

This lesson is a classroom and lab session in which the students will be introduced to hand-operated equipment that is used to make bends in all types of conduit, including electrical metallic tubing (EMT), PVC, and rigid conduit. The lesson will conclude with a performance requirement in which the students will make a 90° bend and back-to-back bends in EMT using a hand bender.

Objectives

Upon completion of this lesson, the student should be able to:

- Identify various types of hand bending equipment and on what type of conduit it is used.

- Apply basic geometric rules in making 90° bends in conduit.

- Make a single 90° bend in EMT.

- Make back-to-back 90° bends in EMT.

Performance Requirements

  - Make a 90° bend and back-to-back 90° bends in EMT using a hand bender.

These tasks partially satisfy module Performance Task 1.

Reading Assignment

  - Module 26107, Sections 1.0.0 thru 2.4.0

Slide Presentation

  - Module 26107, Part 1 (Slides 1 thru 12)

Suggested Equipment/Materials

  - Appropriate personal protective equipment

  - Sufficient sections of ½" EMT for lab work

  - ¾" and 1" EMT pieces for demonstration only

  - Hacksaw

  - ½", ¾", and 1" EMT hand benders

  - ½" rigid conduit hickey bender

  - PVC bending unit

  - PVC bending plug set

  - PVC conduit

  - Wet rags for cooling heated PVC

  - Sufficient sections of ½" rigid metal conduit

  - Sufficient sections of ½" intermediate metal conduit

  - Conduit reamer

  - Tape measure

  - Marker

  - Plastic 90° triangle

  - Torpedo level

Suggested Student Activity

  - Students should identify the different types of conduit and conduit bends that are visible when visiting shopping malls, home improvement stores, and other public buildings. The National Electrical Code®  requires all wiring in commercial buildings to be installed in some type of raceway, unlike residences in which cable may be run within walls and ceiling areas.

Testing

  - Module Review Questions 1 - 6

  - CONTREN Connect Active Figures 6 and 8

Suggested Lab Activities

  - Students must make a single 90° bend in a section of ½” EMT. Use Performance Project 26107-1, Hand Bending a Stub-up.

  - Students must make two back-to-back 90° bends in a section of ½” EMT.

Additional Resources

  - Tom Henry’s Conduit Bending Package. Winter Park, FL: Code Electrical Classes, Inc.

  - Performance Projects: Job Sheets for the Electrical Industry, Level One, New York: Prentice Hall.

  - Contren Connect Electrical 1 Annotated Instructors Guide (available from NCCER).

SLIDE 1

  - Review the objectives to be covered in this module.  Ask the students if they have any questions.

  - Explain that students must obtain a score of 70% on the Module Exam and perform certain tasks to the satisfaction of the instructor to receive credit for the module.

  - Tell the students that safe work practices will be complied with during any of the lab or hands-on assignments, including safety glasses, goggles when necessary, and work gloves.

SLIDE 2

  - Point out that good bending techniques avoid waste and simplify conductor installation.

SLIDE 3

  - Introduce EMT hand benders.

  - Show the students how each bender size is designed for a specific EMT size by placing an EMT sample into the bending head.

  - Explain that it is important to use the correct size bender for the conduit to prevent conduit kinking or creasing.

SLIDE 4

  - Use a ½" bender and a piece of ½" EMT to demonstrate a bend.

SLIDE 5

  - Explain that a hickey bender is designed for bending rigid metal conduit or intermediate metal conduit.

  - Show samples of RMC and IMC, and explain the difference between the two types.

  - Demonstrate how to bend conduit using a hickey.

  - Have the students practice making hickey bends.

SLIDE 6

  - Demonstrate how to bend PVC conduit using a PVC heating unit.

  - Explain to students that if a heating unit is not available, PVC conduit can be bent using a propane hand torch, but this requires practice in order to prevent conduit from burning and/or kinking.

  - At the discretion of the instructor, students may practice bending PVC, but only under strict supervision and safe work practices.

SLIDE 7

  - Demonstrate the use of PVC bending plugs, if available.

  - Explain that plug sets are typically used for bending PVC conduit that is 2 inches in diameter or larger.

SLIDE 8

  - Define the three sides of a 90° triangle using a plastic triangle.

SLIDE 9

  - Explain the formula for finding the circumference of a circle.

  - Have the students practice finding the circumference of a circle based on different radii and diameter measurements.

SLIDE 10

  - Demonstrate how to make a 90° bend in ½" EMT using a hand bender.

  - Have each student measure, mark, and bend a 90° bend in a section of ½" EMT.

SLIDE 11

  - Go over the gain table with the students and explain how it is used to obtain an exact stub length.

  - Demonstrate how to cut a piece of ½" EMT and make a 90° bend using the hand bender so that the measurement of the stub length is 12 inches ± ½-inch.

SLIDE 12

- Discuss the situations in which back-to-back 90° bends might be used.

- Demonstrate how to make back-to-back 90° bends.

- Have each student practice making back-to-back 90° bends.

TESTING and PERFORMANCE

- Have the students answer Review Questions 1 thru 6 for Sections 1.0.0 – 2.4.0. Provide correct answers and remediation at the end of the quiz.

- Have the students complete Active Figures 6 and 8.

- Students must make an acceptable 90° bend and back-to-back 90° bends in ½" EMT using a hand bender. This can be done during the presentation of Slides 10 and 12 or at the end of the lesson. This partially satisfies Performance Task 1.