## Module 02102-05
Plumbing Safety

Lesson 2 of 5: Hazard Communication and Work Zones

This lesson is a classroom session in which the students will learn about hazardous materials found on the job and about how to maintain a safe environment in work zones.  Students will be required to demonstrate the proper use of a ladder to fulfill a Performance Task requirement.

Objectives

Upon completion of this lesson, the student should be able to:

- Identify job-site hazardous work specific to plumbers.

- Demonstrate the proper use of ladders.

- Interpret the NFPA warning diamond.

- Find information on a MSDS.

Performance Requirements

  - Demonstrate the proper use of a ladder.  This corresponds to Performance Task 3.

Reading Assignment

  - Module 02102, Sections 5 & 6

Slide Presentation

  - Module 02102, Part 2 (Slides 21 thru 40)

Suggested Equipment/Materials

  - Samples of MSDS sheets

  - Extension ladder

Suggested Student Activity

  - Provide students with various MSDS sheets.  Have them review the sheets and report anything they see on the sheets that might indicate a hazard and/or the need to wear specialized personal protective equipment.

Testing

  - Module Review Test Questions 1 thru 5 for Section 5.0.0 and Questions 1 thru 10 for Section 6.0.0.

Suggested Lab Activity

  - Students must demonstrate the proper use of a ladder.

Additional Resources

  - www.wfrfire.com/website/articles/ladsafe.htm   (WFR Wholesale Fire & Rescue Ltd.) Ladder safety tips

  - www.MSDS.com  (MSDS Solutions) A free online search of over 3.5 million MSDSs

  - Contren Connect Level 1Plumbng Annotated Instructors Guide (available from the NCCER)

SLIDE 21

  - Review the objectives to be covered in this lesson.  Ask students if they have any questions about the lesson or course objectives.

  - Explain that there are many hazardous materials present at construction sites.

  - Explain that, by law, all workers have a right to know what materials are present and what hazards they present.

  - Explain what HazCom labels are.  Introduce the NFPA warning diamond and explain how to read it.

  - Provide students with examples of NFPA warning diamonds and have them interpret them.

SLIDE 22

  - Introduce the HazCom labels of the National Paint and Coatings Association (NPCA) and the U. S. Dept. of Transportation (DOT).

SLIDE 23

  - Explain what a Material Safety Data Sheet (MSDS) is and the type of information it contains.

  - Explain that each product on a construction site must have an MSDS available.

SLIDES 24 thru 26

  - Use the 3 pages of Figure 18 (3 slides) to describe the information to be found on a typical MSDS.

SLIDE 27

  - Hand out copies of an MSDS for a material that plumbers might encounter, such as a drain cleaner.  Ask students to find specific information about the material.  Concentrate on useful information such as personal protection requirements, handling procedures, and 24-hour emergency-response phone numbers.

  - Explain the purpose of an emergency response plan.  State that all job sites should have an emergency response plan in place.

Have students answer Review Questions for Section 5.0.0.  Provide correct answers and remediation at the end of the quiz.

SLIDE 28

  - Describe what a work zone is and why they are created.

  - Explain that various work zone signs protect workers from hazardous conditions and help them respond to emergencies.

SLIDE 29

  - Describe the purpose of barricades in a work zone.

SLIDE 30

  - Describe the type of information conveyed by work zone signs and tags.

SLIDE 31

  - Describe the characteristics of DANGER signs.

SLIDE 32

  - Describe the characteristics of CAUTION signs.

SLIDE 33

  - Describe the characteristics of INFORMATIONAL signs.

  - Ask the students to identify informational signs they have seen.

SLIDE 34

  - Describe the characteristics of SAFETY signs.

SLIDE 35

  - Describe the characteristics of SAFETY TAGS.

SLIDE 36

  - Describe the signals used on job sites to inform workers of potential dangers.

  - Describe the types of barricades and barriers used to protect against falls through openings in the wall, floor, or ground.

  - Explain that slips, trips, and falls account for 15% of all accidental deaths on construction sites.

  - Describe the hazards that can produce trips or falls on floors and describe methods for reducing those hazards.

  - Describe the hazards that can produce trips or falls through wall openings such as door and windows and describe methods for reducing those hazards.

\

SLIDE 37

  - Describe the hazards that can produce trips or falls on platforms and describe methods for reducing those hazards.

  - Describe the hazards that can produce trips or falls on ramps and runways and describe methods for reducing those hazards.

  - Describe the hazards that can produce falls from ladders and describe methods for reducing those hazards.  Emphasize the importance of keeping the area around the ladder and the ladder itself free from ice or water.

SLIDE 38

  - Review the safe practices for using a ladder contained in section 6.4.6.

  - Demonstrate how to safely secure the top and bottom of a ladder prior to using it.

  - Allow students to practice securing a ladder against a building.

SLIDE 39

  - Review the safe practices for using scaffolding contained in section 6.4.7 of the text.

  - Review the safe practices for walking and working on roofs contained in section 6.4.8 of the text.

  - Describe the use of a toeboard on a pitched roof.

SLIDE 40

  - Review the safe practices for using motor vehicles on the job including personal safety, vehicle safety, using company-owned vehicles, and safe backing.

  - Provide case histories of accidents or injuries caused by the unsafe operation of a motor vehicle.

  - Ask students to relate their own examples of accidents caused by unsafe vehicle operation.

Have students answer Review Questions for Section 6.0.0.  Provide correct answers and remediation at the end of the quiz.

Students must demonstrate the safe use of a ladder. This task corresponds to Performance Task 3.