## Module 02103-05
Plumbing Tools

Lesson 1 of 3: Introduction, and Plumbing Tools, Part One

This lesson is a classroom session in which the students will learn about the care and safe use of tools, measuring and layout tools, leveling tools, and tooth-edged cuttings tools.  Students will identify basic measuring, layout, and cutting tools to fulfill a Performance Task requirement.

Objectives

Upon completion of this lesson, the student should be able to:

- Identify the basic hand and power tools used in the plumbing trade.

- Describe the safety requirements for using plumbing tools.

- Demonstrate how to use a plumber’s level and a line level.

- Identify common tooth-edged cutting tools used by plumbers.

Performance Requirements

  - Identify basic measuring, layout, and cutting tools used by plumbers.  This corresponds to Performance Task 1.

Reading Assignment

  - Module 02103, Sections 1 thru 6

Slide Presentation

  - Module 02103, Part 1 (Slides 1 thru 29)

Suggested Equipment/Materials

  - Various hand and power tools.

Suggested Student Activity

  - Have students obtain the tools shown in Figure 1 and a toolbox to contain them. (Figure 1, Plumber apprentice toolbox).

Testing

  - Module Review Test Questions 1 thru 10 for Sections 2.0.0 – 4.0.0, Questions 1 thru 5 for Section 5.0.0, and Questions 1 thru 5 for Section 6.0.0.

  - CONTREN Connect, Active Figures 7, 18 & 23

Suggested Lab Activity

  - Students must identify basic measuring, layout, and cutting tools used by plumbers.

Additional Resources

  - www.hti.org/0410HTISafety.htm  (Hand Tools Institute) Various publications on the subject of hand tool safety.

  - www.powertoolinstitute.com/teaching.html (Power Tools Institute, Inc.) A variety of teaching materials relating to power tool safety.

  - Contren Connect Level 1Plumbng Annotated Instructors Guide (available from the NCCER)

SLIDE 1

  - Review the objectives for the module.

  - Review the objectives to be covered in this lesson.  Ask students if they have any questions about the lesson or course objectives.

SLIDE 2

  - Explain that plumbers must know how to use a variety of hand and power tools to perform their job.

SLIDE 3

  - Emphasize the importance of properly caring and maintaining hand and power tools.

  - Emphasize the importance of buying high-quality hand and power tools.

  - Review the basic tools that apprentice plumbers should have in their toolbox.

SLIDE 4

  - Emphasize that all tools can become unsafe if not used properly.

  - Review the use of personal protective equipment.

  - Describe the work habits that foster safety on the job.

  - Review the guidelines for safe use and handling of tools in paragraph 3.3.0.

  - Review the guidelines for maintaining tools in paragraph 3.4.0.

  - Ask students to relate incidents they are aware of where someone was injured from using a tool improperly or using an unsafe tool.

SLIDE 5

  - Describe the purpose of measuring and layout tools.

  - Describe the features of a folding rule and demonstrate various applications in which it can be used to measure length.

  - Allow students to practice using the folding rule to measure various dimensions.

SLIDE 6

  - Describe the similarities and differences between a plumber’s rule and a folding rule, including the plumber’s rule use in measuring offsets.

  - Allow students to practice using the plumber’s rule to measure various dimensions.

SLIDE 7

  - Describe the feature and benefits of using a retractable steel tape measure.

  - Allow students to practice using a retractable steel tape to measure various dimensions.

SLIDE 8

  - Describe the feature and benefits of using a hand-cranked retractable tape measure.  Emphasize that this type of tape measure is used to measure longer distances.

  - Allow students to practice using a hand-cranked retractable tape to measure various distances.

SLIDE 9

  - Describe the purpose of squares in the plumbing trade.

  - Describe the purpose of a speed square and demonstrate its use.

  - Allow students to practice using a speed square.

SLIDE 10

  - Describe and demonstrate the use of a combination square.

  - Allow students to practice using the various features of a combination square.

SLIDE 11

  - Describe and demonstrate the use of a framing square.

  - Describe the handling and care of a framing square.

  - Allow students to handle a framing square.

Have students answer Review Questions for Section 2.0.0 – 4.0.0.  Provide correct answers and remediation at the end of the quiz.  Have students complete Active Figure 7.

SLIDE 12

  - Describe the purpose of leveling tools in the plumbing trade.

  - Describe the features of a spirit level and demonstrate how it is used.

  - Allow students to use a spirit level to check the horizontal and vertical level of objects in the classroom.

SLIDE 13

  - Describe the features of a plumber’s level and demonstrate how it is used.  Emphasize differences between it and a standard spirit level such as the ability to measure slope angles.

  - Allow students to use a plumber’s level to check the horizontal and vertical level and the slope of objects in the classroom.

SLIDE 14

  - Describe the features of a torpedo level and demonstrate how it is used.

  - Allow students to use a torpedo level to check the horizontal and vertical level of objects in the classroom.

SLIDE 15

  - Describe the features of a line level and demonstrate how it is used.  Emphasize that this device should not be used if a precise measure of level is required.

  - Allow students to use a line level and string to establish a level line.

SLIDE 16

  - Explain that the leveling devices previously described are inadequate for determining a precision level over a long distance.  Provide examples of these circumstances.

  - Describe the similarities and differences between a builder’s level and a transit level.

  - Describe how they are used to establish level using a graduated rod.

  - Have either a builder’s level or transit level available for students to inspect.

SLIDE 17

  - Describe how a cold beam laser can be used to establish level.

  - Describe the features of a cold beam laser.

  - Describe the precautions that must be taken when using laser levels to avoid eye damage.

  - Review the maintenance and handling procedures for levels and transits.

SLIDE 18

  - Describe the purpose of a plumb bob.

  - Have a plumb bob available to pass around the class.

SLIDE 19

  - Describe and demonstrate the correct procedure for hanging a plumb bob.

SLIDE 20

  - Describe and demonstrate how to use a chalk line to lay out a straight line.

  - Allow students to use a chalk line to lay out straight lines.

Have students answer Review Questions for Section 5.0.0.  Provide correct answers and remediation at the end of the quiz.

SLIDE 21

  - Describe the purpose of tooth-edged cutting tools.

  - Describe the purpose of hacksaws.

  - Have samples of hacksaws available to pass around to students.

SLIDE 22

  - Describe situations where a plumber would have to use a mini-hacksaw.

SLIDE 23

  - Describe the characteristics of hacksaw blades including the kerf.

  - Have samples of hacksaw blades available to pass around the class.

SLIDE 24

  - Explain why and when blades with different teeth-per-inch counts are used in hacksaws.

  - Describe the safety and maintenance instructions for using a hacksaw.

SLIDE 25

  - Describe the features of an electrically-operated reciprocating saw and describe where it would typically be used.

  - Demonstrate the correct operation of a reciprocating saw.

SLIDE 26

  - Describe the features of an electrically-operated portable band saw and describe where it would typically be used.

  - Demonstrate the correct operation of a portable band saw.

SLIDE 27

  - Describe the characteristics of an abrasive saw blade.

  - Describe a demolition saw and explain where it is commonly used.  Stress that this saw can be dangerous and requires the full attention and concentration of the operator when it is in use.

SLIDE 28

  - Describe the characteristics of a chop saw and describe what it is typically used for.

SLIDE 29

  - Thoroughly review the safety guidelines for operating abrasive saws.  See paragraphs 6.4.3 and 6.4.4.

Have students answer Review Questions for Section 6.0.0.  Provide correct answers and remediation at the end of the quiz.  Have students complete Active Figures 18 & 23.

Students must identify basic measuring, layout, and cutting tools.  This task corresponds to Performance Task 1.