## Module 02103-05
Plumbing Tools

Lesson 2 of 3: Plumbing Tools, Part Two

This lesson is a classroom session in which the students will learn about smooth-edged cuttings tools, drilling and boring tools, pipe threading machines, and soldering tools.  Students will identify basic drilling, boring, and reaming tools to fulfill a Performance Task requirement.

Objectives

Upon completion of this lesson, the student should be able to:

- Demonstrate the proper use of plumbing tools.

- Demonstrate the ability to know when and how to select the proper tool(s) for the task.

- Demonstrate how to prepare a surface for tool use.

- Select drill bits based on the material to be drilled.

Performance Requirements

  - Identify basic drilling, boring, and reaming tools used by plumbers.  This corresponds to Performance Task 2.

Reading Assignment

  - Module 02103, Sections 7 thru 10

Slide Presentation

  - Module 02103, Part 2 (Slides 30 thru 52)

Suggested Equipment/Materials

  - Various hand and power tools.

Suggested Student Activity

  - Have students view a video on power tool safety.  At the conclusion of the video, ask each student to relate at least one new fact or piece of information learned from the video.

Testing

  - Module Review Test Questions 1 thru 5 for Section 7.0.0, Questions 1 thru 5 for Section 8.0.0 – 8.4.0, and Questions 1 thru 5 for Section 8.5.0 - 10.0.0.

  - CONTREN Connect, Active Figures 40 & 47

Suggested Lab Activity

  - Students must identify basic drilling, boring, and reaming tools used by plumbers.

Additional Resources

  - www.hti.org/0410HTISafety.htm  (Hand Tools Institute) Various publications on the subject of hand tool safety.

  - www.powertoolinstitute.com/teaching.html (Power Tools Institute, Inc.) A variety of teaching materials relating to power tool safety.

  - Contren Connect Level 1Plumbng Annotated Instructors Guide (available from the NCCER)

SLIDE 30

  - Review the objectives to be covered in this lesson.  Ask students if they have any questions about the lesson or course objectives.

  - Define smooth-edged cutting tools.

  - Describe what a chisel is and how it is used.

  - Describe how plumbers typically use a wood chisel.

  - Have a wood chisel available to pass around to the class.

SLIDE 31

  - Describe and demonstrate how to sharpen the blade of a wood chisel.

SLIDE 32

  - Describe the purpose of cold chisels.

  - . Have a cold chisel available to pass around to the class.

SLIDE 33

  - Describe how a cold chisel is beveled on both sides of the blade.

SLIDE 34

  - Explain that “mushrooming” of a cold chisel head makes it dangerous to use.

  - Describe how to correct a mushroomed chisel head.

SLIDE 35

  - Describe the various types of manual pipe cutters.

  - Describe the purpose of a tube cutter and demonstrate its use.

  - Describe how to mark a tube prior to cutting it.

  - Allow students to mark and cut lengths of scrap copper tubing using a tube cutter.

SLIDE 36

  - Describe and demonstrate the use of a steel pipe cutter.

  - Allow students to mark and cut lengths of scrap steel pipe using a pipe cutter.

SLIDE 37

  - Describe and demonstrate the use of a soil pipe cutter.

  - Allow students to mark and cut lengths of scrap soil pipe using a soil pipe cutter.

  - Describe how to care and maintain tube and pipe cutters.

  - Describe the various types of shovels that plumbers are likely to use.

Have students answer Review Questions for Section 7.0.0.  Provide correct answers and remediation at the end of the quiz.

SLIDE 38

  - Explain that electric drilling and boring tools can increase a plumber’s productivity.

  - Describe and demonstrate the features of a portable electric drill.

  - Have a portable electric drill available to pass around to the class.

SLIDE 39

  - Describe the benefits of using a right-angle drill for drilling holes in close quarters.

  - Have a right-angle electric drill available to pass around to the class.

SLIDE 40

  - Describe the purpose of hammer drills and rotary hammer drills.

  - Have a rotary hammer drill or a hammer drill available to pass around to the class.

SLIDE 41

  - Describe the versatility of pistol drills.

  - Describe the chuck mechanism of a pistol drill and how its size is related to the diameter of the drill bits it can accommodate.

  - Explain the relationship between motor speed, torque and drill bit size.

  - Describe variable-speed and reversing motor drills and the benefits associated with using them.

SLIDE 42

  - Describe the features of cordless drills and the benefits and drawbacks of using them.

  - Have a cordless drill available to pass around the class.

SLIDE 43

  - Describe the various types of drill bits that plumbers are likely to use including twist bits, spade bits, hole-saw bits, self-feeding bits, auger bits, and masonry bits.

  - Have example of each type of drill bit available to pass around to the class.

  - Describe how to properly care for drill bits.

SLIDE 44

  - Describe the purpose of a pipe reamer.

SLIDE 45

  - Describe how a reamer removes the burr from the inside of a pipe.  Caution against over-reaming.

SLIDE 46

  - Describe and demonstrate how a pencil reamer is used to remove burrs from copper tubing.

  - Have a variety of reamers available to pass around to the class.

Have students answer Review Questions for Sections 8.0.0 – 8.4.0.  Provide correct answers and remediation at the end of the quiz.  Have students complete Active Figure 40.

SLIDE 47

  - Explain that dies are used to cut male threads on steel pipe so that pipe sections can be screwed together using threaded female fittings.

  - Have samples of pipe cutting dies available to pass around to students.

SLIDE 48

  - Describe and demonstrate how the die tools are inserted into the ratchet stock.

  - Allow students to install die tools into a ratchet stock.

SLIDE 49

  - Describe and demonstrate how operate a die and ratchet stock to thread pipe.

  - Allow students to thread pipe using a die and ratchet stock.

SLIDE 50

  - Describe the features of an electric pipe-threading machine and demonstrate how to operate it to thread pipe.

  - Under your supervision, allow students to thread pipe using an electric pipe-threading machine.

SLIDE 51

  - Describe the process of soldering to join copper pipe.

  - Explain that gases such as propane and acetylene can be used to provide heat for soldering.

  - Review safety precautions for working with various fuel gases used for soldering.

  - Describe how a carbon-arc torch can be used for soldering.

SLIDE 52

  - Explain that a striker must be used to light a torch.  Never use a match or an open flame.

Have students answer Review Questions for Sections 8.5.0 – 10.0.0.  Provide correct answers and remediation at the end of the quiz.  Have students complete Active Figure 47.

Students must identify basic drilling, boring, and reaming tools.  This task corresponds to Performance Task 2.