## Module 02110-05
Corrugated Stainless Steel Tubing

Lesson 1 of 1: Corrugated Stainless Steel Tubing

This lesson is a combination classroom and lab session in which the students will learn about corrugated stainless steel tubing (CSST).  Students will simulate and/or demonstrate how to join corrugated stainless steel tubing to fulfill a Performance Task requirement.

Objectives

Upon completion of this lesson, the student should be able to:

- Identify the common manufacturers of CSST.

- Recognize proper and improper applications of CSST.

- Identify the various techniques uses in hanging and supporting CSST.

- Explain how to properly measure, cut, and join CSST.

- Identify the material properties, storage, and handling requirements for CSST.

Performance Requirements

  - Simulate and/or demonstrate how to join CSST. This corresponds to Performance Task 1.

Reading Assignment

  - Module 02110, All Sections

Slide Presentation

  - Module 02110, Part 1 (Slides 1 thru 25)

Suggested Equipment/Materials

  - Tools and materials required to measure, cut, and join CSST.

Suggested Student Activity

  - Have students do an on-line search of CSST manufactures to find and retrieve application and installation information.

Testing

  - Module Review Test Questions 1 thru 18 for Section 1.0.0 – 5.0.0.

  - CONTREN Connect, Active Figure 9

  - Module Trade Terms Quiz

  - Module Examination

Suggested Lab Activity

  - Students must simulate and/or demonstrate how to join CSST.

Additional Resources

  - www.gastite.com/page.php (Gastite) Downloadable Size-It software used to design the layout of a CSST gas supply system.

  - www.mtlfab.com/metalfab/content/view/full/386 (Metal-Fab, Inc.) Design guide and installation manual for CSST.

  - Contren Connect Level 1Plumbng Annotated Instructors Guide (available from the NCCER)

SLIDE 1

  - Review the objectives to be covered in this lesson.  Ask students if they have any questions about the lesson objectives.

SLIDE 2

  - Describe what corrugated stainless steel tubing (CSST) is and what it is used for.

SLIDES 3 and 4

  - Describe the advantages of using CSST when installing gas lines.  See Slides 3 & 4 (Figures 1 & 2).

  - Have samples of CSST available to pass around the class.

SLIDE 5

  - Explain that CSST is made by several manufacturers and sold as a system.

  - Explain that tubing and fittings are not interchangeable between different manufacturers.

  - Have samples of different manufacturer’s tubing and fittings available to show their differences.

SLIDE 6

  - Discuss the various markings found on CSST.  Have samples of marked tubing available to pass around the class.

  - Discuss the importance of properly sizing CSST piping systems.

  - Explain that installers must be trained before they can install CSST systems.

SLIDE 7

  - Describe the pressures typically encountered in gas piping systems.

  - Explain that gas regulators are often required to lower the pressure in gas piping systems.

  - Have a gas regulator available to pass around the class.

SLIDE 8

  - Explain that ball valves or gas cocks are required in gas piping systems to shut off the flow of gas.

  - Have a gas cock and/or ball valve available to pass around the class.

SLIDE 9

  - Explain how individual appliances can be supplied with gas from CSST branches off the main CSST gas line.

SLIDE 10

  - Explain how multiple appliances can be supplied with gas from CSST branches off the main CSST gas line.

SLIDE 11

  - Explain how manifolds can be used to connect multiple appliances to the main CSST gas line.  See slides 11 & 12 (Figures 8 & 9).

SLIDE 12

No notes

SLIDE 13

  - Discuss the cutters used to cut CSST.

SLIDE 14

  - Describe and demonstrate how to cut CSST with tube cutters.

  - Allow students to practice cutting CSST with tube cutters.

  - Explain that only certain fittings and materials can be used to connect CSST to appliances and other pipe.

SLIDE 15

  - Describe and demonstrate how to connect CSST to an appliance with a straight fitting.  See slides 15-17 (Figure 12 & two-part figure 13).

  - Allow students to practice making a connection with a straight fitting.

SLIDE 16

No Notes

SLIDE 17

No notes

SLIDE 18

  - Describe the use of stub-outs for connecting to hidden pipes.

  - Have a stub-out available to pass around the class.

SLIDE 19

  - Describe the use of regulator stub-outs for connecting to regulators.

  - Have a regulator stub-out available to pass around the class.

SLIDE 20

  - Explain that CSST sections can be connected together using CSST couplings.

  - Have CSST couplings available to pass around the class.

SLIDE 21

  - Explain that CSST sections can be connected together using CSST tees

  - Have CSST tees available to pass around the class.

SLIDE 22

  - Describe the do’s and don’ts of hanging and running CSST.

  - Explain that CSST can be supported with pipe clamps on a strut (Slide 22, Figure 18) or with pipe straps (Slide 23, Figure 19).

  - Have samples of pipe clamps and pipe straps available to pass around the class.

SLIDE 23

No Notes

SLIDE 24

  - Describe the steps to take to protect CSST when it is concealed within walls or buried.

SLIDE 25

  - Describe the procedures for inspecting and testing installed CSST systems.

Have students answer Review Questions for Sections 1.0.0 - 5.0.0.  Provide correct answers and remediation at the end of the quiz.  Have students complete Active Figure 9.

Have students complete the module Trade Terms Quiz

Students must simulate and/or demonstrate how to join corrugated stainless steel tubing.  This task corresponds to Performance Task 1.

At the completion of this session, provide a module review and then administer the Module Examination.  Students must achieve a score of at least 70% and have satisfactorily completed all lab exercises and/or Performance Testing requirements to receive credit for the module.