## Module 02112-05
Introduction to Drain, Waste, and Vent (DWV) Systems

Lesson 2 of 2: Vents, Drains, and Fittings, and DWV System Design

This lesson is a classroom session in which the students will learn about drain, waste, and vent (DWV) systems.  Students will sketch a simple DWV system, label the components, and size the pipe to fulfill a Performance Task requirement.

Objectives

Upon completion of this lesson, the student should be able to:

- Identify the various types of DWV fittings and describe their applications.

- Describe the types of materials used for DWV pipes and fittings.

- Describe the major components of a conventional septic system.

- Identify the significant code and health issues, violations, and consequences related to DWV systems.

Performance Requirements

  - Sketch a simple DWV system, label the components, and size the pipe. This corresponds to Performance Task 1.

Reading Assignment

  - Module 02112, Sections 5 thru 13

Slide Presentation

  - Module 02112, Part 2 (Slides 17 thru 46)

Suggested Equipment/Materials

  - Samples of DWV fittings

Suggested Student Activity

  - Have students visit a residential construction site to observe a septic system being installed.  Have students write a brief report on what they observed.

Testing

  - Module Review Test Questions 1 thru 5 for Sections 5.0.0 – 6.0.0, Questions 1 thru 10 for Section 7.0.0, and Questions 1 thru 6 for Sections 8.0.0 – 13.0.0

  - CONTREN Connect, Active Figures 14 & 39

  - Module Trade Terms Quiz

  - Module Examination

Suggested Lab Activity

  - Students must sketch a simple DWV system, label the components, and size the pipe.

Additional Resources

  - www.snydernet.com/onsite_septic_water_tank_installation.htm (Snyder Industries, Inc.) Instructions for installing septic tanks.

  - www.nibco.com/cms.do?id=28pId=57 (Nibco) A catalog of DWV fittings.

  - Contren Connect Level 1Plumbng Annotated Instructors Guide (available from the NCCER)

SLIDE 17

  - Review the objectives to be covered in this lesson.  Ask students if they have any questions about the lesson objectives.

  - Explain the purpose of vents in a DWV system.

  - Discuss the placement of vents near traps.

SLIDE 18

  - Discuss issues related to the sizing of drains and vents.

Have students answer Review Questions for Sections 5.0.0 – 6.0.0.  Provide correct answers and remediation at the end of the quiz.  Have students complete Active Figure 14.

SLIDE 19

  - Describe what drainage fittings are.

  - Describe the materials used to make DWV fittings.  Explain that not all fittings are available in all materials.

SLIDE 20

  - Explain that DWV fittings are made to allow for a smooth flow of wastes.

  - Describe how fittings should be installed to facilitate smooth flow.

SLIDE 21

  - Explain that purpose of sanitary fittings.

  - Describe the purpose of bends and describe various types of bends available.

  - Have samples of bends available to pass around the class.

SLIDE 22

  - Describe the purpose of bends with heel and side inlets.

  - Have samples of bends with heel and/or side inlets available to pass around the class.

SLIDE 23

  - Describe how to determine if a side inlet bend is a right side or left side inlet.

SLIDE 24

  - Explain why long sweep and short sweep ¼ bends are used at the base of DWV stacks.

  - Have long and short sweep ¼ bends available to pass around the class.

SLIDE 25

  - Describe the purpose of double ¼ bends.

  - Have a double ¼ bend available to pass around the class.

SLIDE 26

  - Describe the purpose of vent ells.

  - Have a vent ell available to pass around the class.  Explain that these ells are only available in plastic.

SLIDE 27

  - Describe the purpose of adapters.

  - Have various adapters available to pass around the class.

SLIDE 28

  - Describe the purpose of cleanouts and cleanout adapters.

  - Have samples of cleanout adapters available to pass around the class.

SLIDE 29

  - Describe the purpose of sanitary tees.  Discuss their usage restrictions.

  - Have a sanitary tee available to pass around the class.

SLIDE 30

  - Describe the purpose of vent tees.  Discuss their usage restrictions.

  - Have a vent tee available to pass around the class.

SLIDE 31

  - Describe the purpose of test tees.

  - Have a test tee available to pass around the class.

SLIDE 32

  - Describe the purpose of sanitary wyes (Slide 32, Figure 28) and sanitary upright wyes (Slide 33, Figure 29).

  - Have a sanitary wye available to pass around the class.

SLIDE 33

No Notes

SLIDE 34

  - Describe the purpose of vent branches (Slide 34, Figure 30) and inverted wyes (Slide 35, Figure 31).

  - Have a vent branch and/or inverted wye available to pass around the class.

SLIDE 35

No Notes

SLIDE 36

  - Describe the purpose of sanitary combinations.

  - Have a sanitary combination available to pass around the class.

SLIDE 37

  - .  Explain that combination fittings can be field-fabricated.

SLIDE 38

  - Explain that there are a number of miscellaneous fittings available for DWV systems.

  - Describe the purpose of sanitary increasers.

  - Have a sanitary increaser available to pass around the class.

SLIDE 39

  - Describe the purpose of sanitary increasers.

  - Have a sanitary increaser available to pass around the class.

Have students answer Review Questions for Section 7.0.0.  Provide correct answers and remediation at the end of the quiz.

SLIDE 40

  - Explain the terms grade and slope as they relate to DWV systems.

  - Explain how important grade is to the proper functioning of DWV systems.

SLIDE 41

  - Explain what a building drain is and what purpose it serves.

  - Describe the various components of a building drain.

  - Describe the purpose of cleanouts.

SLIDE 42

  - Describe the characteristics of the building sewer and the materials it is typically made of.

SLIDE 43

  - Describe the characteristics of a sewer main.

SLIDE 44

  - Describe the characteristics of a municipal waste treatment plant and describe how it operates.

SLIDE 45

  - Describe the characteristics of a private waste disposal system.

  - Describe the major components of a septic system.

  - Ask students if any of them live in a home that uses a septic system.  If so, ask them about any problems they have experienced with it and actions taken to correct the problem.

SLIDE 46

  - Discuss various code and health issues associated with DWV systems.

  - Ask students to relate any incidents that they are aware of where a faulty DWV system caused a public health or safety problem.

Have students answer Review Questions for Sections 8.0.0 – 13.0.0.  Provide correct answers and remediation at the end of the quiz.  Have students complete Active Figure 39.

Have students complete the module Trade Terms Quiz.

Students must sketch a simple DWV system, label its components, and size the pipe.  This task corresponds to Performance Task 1.

At the completion of this session, provide a module review and then administer the Module Examination.  Students must achieve a score of at least 70% and have satisfactorily completed all lab exercises and/or Performance Testing requirements to receive credit for the module.