## Module 02109-05
Carbon Steel Pipe and Fittings

Lesson 2 of 2: Measuring, Cutting, Threading, and Joining Steel Pipe, and Hangers and Supports

This lesson is a combined classroom and lab session in which the students will learn how to measure, cut, thread, and join carbon steel pipe, and how to properly hang and support them.  Students will identify the hazards and safety precautions associated with carbon steel piping, measure, cut, thread, and join carbon steel piping, and identify the various techniques used in hanging and supporting steel piping to fulfill Performance Task requirements.

Objectives

Upon completion of this lesson, the student should be able to:

- Measure, cut, thread, and join carbon steel piping.

- Describe safety precautions for cutting and threading carbon steel piping.

- Identify the various techniques uses in hanging and supporting carbon steel piping.

Performance Requirements

  - Measure, cut, thread, and join carbon steel piping. This corresponds to Performance Task 3.

  - Identify the hazards and safety precautions associated with carbon steel piping. This corresponds to Performance Task 4.

  - Identify the various techniques used in hanging and supporting carbon steel piping.  This corresponds to Performance Task 5.

Reading Assignment

  - Module 02109, Sections 4 and 5

Slide Presentation

  - Module 02109, Part 2 (Slides 20 thru 50)

Suggested Equipment/Materials

  - Lengths of carbon steel pipe and fittings.

  - Tools and materials required to measure, cut, thread, and join carbon steel pipe

  - Assorted hanger and support hardware.

Suggested Student Activity

  - Under your supervision, have students measure, cut, thread, and join carbon steel piping.

Testing

  - Module Review Test Questions 1 thru 5 for Section 4.0.0, and Questions 1 thru 5 for Section 5.0.0.

  - Module Trade Terms Quiz

  - CONTREN Connect, Active Figures 35 & 36

  - Module Examination

Suggested Lab Activity

  - Students must measure, cut, thread, and join carbon steel piping, identify the hazards and safety precautions associated with carbon steel piping, and identify the various techniques used in hanging and supporting steel piping.

Additional Resources

  - www.rigid.com/tools/training-videos/index.htm (Ridge Tool Co.)  Downloadable training videos on pipe threading machine operation

  - www.oatey.com (Oatey) MSDS sheets for pipe-joint compound and thread cutting oil

  - Contren Connect Level 1Plumbng Annotated Instructors Guide (available from the NCCER)

SLIDE 20

  - Review the objectives to be covered in this lesson.  Ask students if they have any questions about the lesson or course objectives.

  - Explain that special methods are used to cut and thread carbon steel pipe.

  - Explain why thread engagement must be accounted for when measuring pipe prior to cutting it.

SLIDE 21

  - Describe and demonstrate the various ways that threaded pipe can be measured.

SLIDE 22

  - Describe and demonstrate how to cut carbon steel pipe with a pipe cutter.

  - Allow students to practice cutting carbon steel pipe with a pipe cutter.

SLIDE 23

  - Describe and demonstrate how to ream carbon steel pipe with a pipe reamer.

  - Allow students to practice reaming carbon steel pipe with a pipe reamer.

SLIDE 24

  - Describe and demonstrate how to select and install dies and how to thread carbon steel pipe with a hand pipe threader.

  - Allow students to practice threading carbon steel pipe with a hand pipe threader.

SLIDE 25

  - Describe and demonstrate how to thread carbon steel pipe with a power drive threader.

  - Review the safety precautions associated wit use of a powered pipe threader.

  - Allow students to practice threading carbon steel pipe with a power drive threader.

SLIDE 26

  - Describe how to cut, ream, and thread pipe with a power threading machine.  Explain that this machine is normally used when large quantities of pipe must be threaded.

  - Emphasize the importance of using cutting oil during cutting and threading operations.

SLIDE 27

  - Describe and demonstrate how to hold pipe during threading with a yoke vise

  - Allow students to practice using the vise when threading pipe.

SLIDE 28

  - Describe and demonstrate how to hold pipe during threading with a chain vise

  - Allow students to practice using the vise when threading pipe.

SLIDE 29

  - Describe and demonstrate how to use the various pipe wrenches.  Describe the advantages of each type of wrench and where each one is typically used (Slides 29 thru 34, Figures 24 thru 29)

  - Allow students to practice using the various wrenches.

SLIDE 30

No notes

SLIDE 31

No notes

SLIDE 32

No notes

SLIDE 33

No notes

SLIDE 34

No notes

SLIDE 35

  - Describe and demonstrate how to cut threads using a hand die and stock.

  - Allow students to practice cutting threads using a hand die and stock.

SLIDE 36

  - Describe and demonstrate how to cut threads using a power-threading machine.

  - Allow students to practice cutting, reaming, and threading pipe with a power threading machine under your direct supervision.

SLIDE 37

  - Describe and demonstrate the steps for joining threaded pipe.

  - Show how to properly apply pipe joining compound and tape.

  - Allow students to practice joining threaded pipe.

SLIDE 38

  - Describe the pipe grooving process and explain the differences between roll grooving and cut grooving.

SLIDE 39

No notes

SLIDE 40

  - Describe the method used to join grooved pipe.

  - If equipment is available, demonstrate how to join grooved pipe and allow students to practice joining grooved pipe.

Have students answer Review Questions for Section 4.0.0. Provide correct answers and remediation at the end of the quiz.  Have students complete Active Figure 35.

Students must measure, cut, thread, and join carbon steel piping, and identify the hazards and safety precautions associated with carbon steel piping. These tasks correspond to Performance Tasks 3 and 4 respectively.

SLIDE 41

  - Explain why carbon steel pipes must be properly supported.

  - Describe the factors affecting the selection of hangers and supports.

  - Have samples of various horizontal pipe hangers available to pass around the class.

SLIDE 42

  - Describe some of the pipe supports used with wood frames.

  - Have samples available to pass around the class.

SLIDE 43

  - Describe the use of riser clamps to support vertical pipes.

  - Have a riser clamp available to pass around the class.

SLIDE 44

  - Describe the use of universal pipe clamps (Slide 44, Figure 39) and pipe channels (Slides 45 & 46, two-part Figure 40) to support pipes.

  - Have examples of this equipment available to pass around the class.

SLIDE 45

No notes

SLIDE 46

No notes

SLIDE 47

  - Describe the purpose of connectors and show how they are used with threaded rods.

  - Have connectors and threaded rods available to pass around the class.

SLIDE 48

  - Describe the types of anchors used to secure pipe supports to concrete and brick walls.

  - Have examples of anchors available to pass around the class.

SLIDE 49

  - Describe how clamps are used to secure pipe supports to structural members such as girders and beams.

  - Have examples of these clamps available to pass around the class.

SLIDE 50

  - Describe the use of riser clamps to support vertical pipes in wood frame structures.

Have students answer Review Questions for Section 5.0.0.  Provide correct answers and remediation at the end of the quiz.  Have students complete Active Figure 36.

Have students complete the module Trade Terms Quiz.

Students must Identify the various techniques used in hanging and supporting carbon steel piping.  This task corresponds to Performance Task 5.

At the completion of this session, provide a module review and then administer the Module Examination.  Students must achieve a score of at least 70% and have satisfactorily completed all lab exercises and/or Performance Testing requirements to receive credit for the module.