## Module 02113-05
Introduction to Water Distribution Systems

Lesson 1 of 1: Introduction, Supply and Distribution, Cross-Connection, and Building Distribution

This lesson is a classroom session in which the students will learn about water distribution systems.  Students will identify the major components of a water distribution system, and describe the function of each component to fulfill a Performance Task requirement.

Objectives

Upon completion of this lesson, the student should be able to:

- Identify the major components of a water distribution system, and describe the function of each component.

- Describe the process in which water is distributed in municipal, residential, and private water systems.

- Explain the relationships among components of a water distribution system.

- Describe the various stages that water passes through at a municipal water treatment plant.

- Describe some of the hazards associated with cross-connections.

Performance Requirements

  - Identify the major components of a water distribution system, and describe the function of each component. This corresponds to Performance Task 1.

Reading Assignment

  - Module 02113, All Sections

Slide Presentation

  - Module 02113, Part 1 (Slides 1 thru 34)

Suggested Equipment/Materials

  - Various valves and fittings used in a water distribution system.

Suggested Student Activity

  - Have students visit a municipal water treatment plant to observe its operation.  Have students write a brief report on what they observed.

Testing

  - Module Review Test Questions 1 thru 5 for Sections 3.0.0, Questions 1 thru 10 for Section 4.0.0, Questions 1 thru 10 for Sections 5.0.0, and Questions 1 thru 10 for Sections 6.0.0 – 7.0.0

  - CONTREN Connect, Active Figures 2, 4 & 10

  - Module Trade Terms Quiz

  - Module Examination

Suggested Lab Activity

  - Students must identify the major components of a water distribution system, and describe the function of each component.

Additional Resources

  - Introduction to Potable Water Treatment Processes, Malden, MA, Blackwell Publishing.

  - www.lifewater.ca/ndexdril.htm (Lifewater Canada) An on-line tutorial on how to build a safe drinking water well.

  - Contren Connect Level 1Plumbng Annotated Instructors Guide (available from the NCCER)

SLIDE 1

  - Review the objectives to be covered in this lesson.  Ask students if they have any questions about the lesson objectives.

SLIDE 2

  - Describe the water distribution process from the source through treatment and distribution to the home.

SLIDE 3

  - Describe the characteristics of the different types of wells drilled to obtain water.

  - Ask students if their domestic water is obtained directly from a private well.  If so, ask them to describe the type of well used.

SLIDE 4

  - Explain why water must be treated before it is considered potable.

  - Describe how water is treated in private water systems.

SLIDE 5

  - Describe the steps for treating water in a municipal water system.

SLIDE 6

  - Describe the methods used to maintain pressure in municipal water systems including the use of booster pumps (Slide 6, Figure 5) and pressure regulators (Slide 7, Figure 6).

SLIDE 7

Have students answer Review Questions for Section 3.0.0.  Provide correct answers and remediation at the end of the quiz.  Have students complete Active Figures 2 and 4.

SLIDE 8

  - Describe the materials used to construct water lines, giving advantages and disadvantages of each type of material.

  - Explain what galvanic corrosion is and what causes it.

SLIDE 9

  - Describe the supply piping and pump arrangement in a private water supply system.

SLIDE 10

  - Describe the characteristics, similarities and differences between, and locations of corporation stops (Slide 10, Figure 8) and curb stops (Slide 11, Figure 9).

SLIDE 11

No Notes

SLIDE 12

  - Explain that the curb stop is located in a curb box.  A special key is used to turn off the curb stop.

SLIDE 13

  - Explain that the curb box is usually marked with a metal marker.

SLIDE 14

  - Describe the location of the water meter.

Slide 15

  - Explain that a meter shutoff valve is installed ahead of the water meter to shut off service to the building.

SLIDE 16

  - Explain that the building main shutoff valve is located after the water meter and inside the building, allowing water to be shut off during construction or for emergency repairs.

Have students answer Review Questions for Section 4.0.0.  Provide correct answers and remediation at the end of the quiz.  Have students complete Active Figure 10.

SLIDE 17

  - Describe what backflow is and the undesirable results it can bring.

  - Describe how a vacuum breaker on a hose bibb acts as a backflow preventer.

  - Explain that codes specify what type of backflow preventer to use in a particular installation.

  - Have students review Table 1 in their textbook for a better understanding of the types of backflow preventers used in different applications.

SLIDE 18

  - Explain that some backflow preventers can protect against both backpressure and back siphonage, while others can handle only one type of backflow.

  - Describe the characteristics of the five most commonly used backflow preventers.

SLIDE 19

  - Explain that valves and faucets are used to regulate the flow of water in water distribution systems.

  - Explain the following valve terms: straight-through flow, full flow, and throttled flow.

SLIDE 20

  - Describe the characteristics of gate valves and their use in a water supply system.

  - Have a gate valve available to pass around the class.

SLIDE 21

  - Describe the characteristics of globe valves and their use in a water supply system.

  - Point out that globe valves, unlike gate valves, can be used to modulate flow.

  - Have a globe valve available to pass around the class.

SLIDE 22

  - Describe the characteristics of angle valves and their use in a water supply system.

  - Have an angle valve available to pass around the class.

SLIDE 23

  - Describe the characteristics of ball valves and their use in a water supply system.

  - Have a ball valve available to pass around the class.

SLIDE 24

  - Describe characteristics of check valves and their use in a water supply system.  Describe and compare the operation of ball-check valves (Slide 24, Figure 22), swing-check valves (Slide 25, Figure 23), and lift-check valves (Slide 26, Figure 24).

  - Have samples of the three check valves available to pass around the class.

SLIDE 25

No Notes

SLIDE 26

No Notes

SLIDE 27

  - Describe the characteristics of pressure regulator valves and their use in a water supply system.

  - Have a pressure regulator valve available to pass around the class.

SLIDE 28

  - Describe the characteristics of supply stop valves and their use in a water supply system.

  - Have a supply stop valve available to pass around the class.

SLIDE 29

  - Describe the characteristics of temperature and pressure (T & P) valves and their use in a water supply system.

  - Have a T & P valve available to pass around the class.

Have students answer Review Questions for Section 5.0.0.  Provide correct answers and remediation at the end of the quiz.

SLIDE 30

  - Describe how and where to locate the water heater, water softener, hose bibs, and fixtures (Slides 30 & 31, Figures 28 & 29).

SLIDE 31

  - Explain that the DWV system is typically installed first, with the water supply piping installed in relation to the DWV system.

SLIDE 32

  - Describe how to construct fixture risers and how to connect them to backing boards.

  - Describe how risers are connected to feeder lines.

SLIDE 33

  - Discuss the purpose of water hammer arresters and why they are used in a water supply system.

  - Have a water hammer arrester available to pass around the class.

  - Discuss the various factors that affect the size of the main water supply line.

  - Have students study Tables 2 and 3 in their textbook for a better understanding of the factors that affect water supply line size.

SLIDE 34

  - Explain how tempering valves are used in hot water supply systems to prevent scalding.

  - Have a tempering valve available to pass around the class.

Have students answer Review Questions for Sections 6.0.0 – 7.0.0.  Provide correct answers and remediation at the end of the quiz.

Have students complete the module Trade Terms Quiz

Students must identify the major components of a water distribution system, and describe the function of each component.  This task corresponds to Performance Task 1.

At the completion of this session, provide a module review and then administer the Module Examination.  Students must achieve a score of at least 70% and have satisfactorily completed all lab exercises and/or Performance Testing requirements to receive credit for the module.