Understanding pH

Pure (neutral) water has a pH around 7 at 25 °C (77 °F); this value varies with temperature. When an acid is dissolved in water, the pH will be less than 7 (if at 25 °C (77 °F)). When a base, or alkali, is dissolved in water, the pH will be greater than 7 (if at 25 °C (77 °F)). A solution of a strong acid, such as hydrochloric acid, at concentration 1 mol/L has a pH of 0. A solution of a strong alkali, such as sodium hydroxide, at concentration 1 mol/L, has a pH of 14. Thus, measured pH values will lie mostly in the range 0 to 14. Since pH is a logarithmic scale, a difference of one pH unit is equivalent to a tenfold difference in hydrogen ion concentration.

An approximate measure of pH may be obtained by using a pH indicator. A pH indicator is a substance that changes color around a particular pH value. It is a weak acid or weak base and the color change occurs around 1 pH unit either side of its acid dissociation constant, or pKa, value. For example, the naturally occurring indicator litmus is red in acidic solutions (pH<7 at 25 °C (77 °F)) and blue in alkaline (pH>7 at 25 °C (77 °F)) solutions. Universal indicator consists of a mixture of indicators such that there is a continuous color change from about pH 2 to pH 10. Universal indicator paper is simple paper that has been impregnated with universal indicator.

| Universal indicator components |  |  |  |
| --- | --- | --- | --- |
| Indicator | Low pH color | Transition pH range | High pH color |
| Thymol blue (first transition) | Red | 1.2 – 2.8 | Yellow |
| Methyl red | Red | 4.4 – 6.2 | Yellow |
| Bromothymol blue | Yellow | 6.0 – 7.6 | Blue |
| Thymol blue (second transition) | Yellow | 8.0 – 9.6 | Blue |
| Phenolphthalein | Colorless | 8.3 – 10.0 | Fuchsia |

A solution whose pH is 7 (at 25 °C (77 °F)) is said to be neutral, that is, it is neither acidic nor basic. Water is subject to a self-ionization process.

H2O H+ + OH−

The dissociation constant, KW, has a value of about 10−14, so, in neutral solution of a salt, both the hydrogen ion concentration and hydroxide ion concentration are about 10−7 mol dm−3. The pH of pure water decreases with increasing temperatures. For example, the pH of pure water at 50 °C is 6.55. Note, however, that water that has been exposed to air is mildly acidic. This is because water absorbs carbon dioxide from the air, which is then slowly converted into carbonic acid, which dissociates to liberate hydrogen ions:

CO2 + H2O  H2CO3  HCO3− + H+

A pH meter is an electronic instrument used for measuring the pH (acidity or alkalinity) of a liquid (though special probes are sometimes used to measure the pH of semi-solid substances). A typical pH meter consists of a special measuring probe (a glass electrode) connected to an electronic meter that measures and displays the pH reading.

The pH probe measures pH as the activity of the hydrogen cations surrounding a thin-walled glass bulb at its tip. The probe produces a small voltage (about 0.06 volt per pH unit) that is measured and displayed as pH units by the meter.

- Calibration and use of pH meter

For very precise work the pH meter should be calibrated before each measurement. For normal use calibration should be performed at the beginning of each day. The reason for this is that the glass electrode does not give a reproducible reading over longer periods of time. Calibration should be performed with at least two standard buffer solutions that span the range of pH values to be measured. For general purposes buffers at pH 4 and pH 10 are acceptable. The pH meter has one control (calibrate) to set the meter reading equal to the value of the first standard buffer and a second control (slope) which is used to adjust the meter reading to the value of the second buffer. A third control allows the temperature to be set. For more precise measurements, a three buffer solution calibration is preferred. As pH 7 is essentially, a "zero point" calibration (akin to zeroing or taring a scale or balance), calibrating at pH 7 first, calibrating at the pH closest to the point of interest ( e.g. either 4 or 10) second and checking the third point will provide a more linear accuracy to what is essentially a non-linear problem. Some meters will allow a three point calibration and that is the preferred scheme for the most accurate work. After each single measurement, the probe is rinsed with distilled water or deionized water to remove any traces of the solution being measured, blotted with a scientific wipe to absorb any remaining water which could dilute the sample and thus alter the reading, and then quickly immersed in another storage solution (KCL-not water).