555 Lab Worksheet

Using a breadboard and the provided components, construct the following timer circuit:

The supply voltage may be +5 to +15 volts. Using the calculations provided in the 555 Operation Sheet, the LED should flash about once per second. Answer the following questions:

- Count the number of flashes of the LED over a 20 second period of time. Number of flashes = __________________

- What would you calculate for the number of flashes per minute? _________

- Using an oscilloscope, observe and record the following:

  - LED on-time = ___________

  - LED off-time = ___________

  - Total of LED on and off time = __________________

- The calculated total time was 0.98 seconds. Does this differ from the amount observed in 3c? ________________ Why? ___________________

______________________________________________________

- What factors might affect the difference between the calculated and measured times? ________________________________________

- What would happen if you changed the supply voltage (for example, instead of 5VDC, a change is made to apply 10VDC)? __________________ Would you expect a change in the frequency of operation? ___________

Why or why not? ____________________________________________