Using Watt’s Law Practice

Solve the following problems using Ohm’s Law and Watt’s Law as appropriate.

- A 12 volt battery is providing a circuit current of 25 ma. If this is the only power source in the circuit, what is the equivalent circuit resistance? ___480Ω____________________________

- In the above circuit, how much power does the battery provide to the circuit? 0.3 Watts______

- Assume you have a flashlight that uses four 1.5 volt batteries in series (total of 6 volts) and a bulb that is rated at 2 watts. How much current will the bulb draw from the circuit? __0.33 amperes

- You have a 1500 watt electric space heater that operates on standard household voltage of 120 VAC. How much current flows through the heater during operation? ____12.5 amperes_________________

- If you have a ½ watt resistor of 10 Ω, what would you expect to happen if you placed it across the terminals of a 12 volt battery? It woul d fail or overheat____ Why?___The resistance across the battery would draw 1.2 amps and the power required would be  14.4 watts! This exceeds the design value of the resistor._________________

- A series circuit consisting of 3 resistors (68Ω, 82Ω, 150Ω) and a battery draws a current of 0.7 ampere. What must be the voltage of the battery? ____210 volts______________ What is the power drain on the battery by the circuit? ________147 watts______________________

- A parallel circuit containing 2 resistors (390 kΩ and 150 KΩ) and a battery draws a current of 1 ½ ma. What must be the voltage of the battery? ____16.35 volts______ What is the power drain on the battery by the circuit? __________25 mw___________________

- The unit of power is known as ___watt______________.

- The unit of energy is known as ___joule______________.

- In a simple parallel circuit consisting of a 20 volt battery and 2 resistors, each with a value of 100Ω, how much power is needed from the battery when the circuit is in operation? 8 watts

Extra Credit: If a bulb is rated at 75 watts, would you expect it to produce 75 joules of light energy every second when in operation?_____No_____________  Why/Why not? ______Although 75 joules of energy would be expended, not all of it will be light energy. A good amount of the expended energy will be in the form of heat.