Materials Worksheet

We learned earlier about a wide variety of types of materials used in fasteners. Today, we are going to review in close detail the three types of stainless steels: austenitic, martensitic or ferritic. You will be provided in one column a description of what is needed for the product or basic information about the material and then you will check off under austenitic, martensitic or ferritic what material you recommend as a match. Information in the Fastenal catalog will be of benefit to you on this assignment.

|  | Austenitic | Martensitic | Ferritic |
| --- | --- | --- | --- |
| Offers high strength at reasonably elevated temperatures |  |  |  |
| Should only be used in mild corrosive environments |  |  |  |
| Commonly used for wire products such as springs, screens, cables |  |  |  |
| Common material for flat washers |  |  |  |
| Should not be used in situations of high corrosion resistance requirements |  |  |  |
| Contain higher amounts of chromium and nickel than the other types |  |  |  |
| In crevices and shielded areas, there might not be enough oxygen to maintain the passive oxide film and crevice corrosion might occur |  |  |  |
| Capable of being heat treated in such a way that the martensite is the prime microconstituent |  |  |  |
| This type of steel is magnetic, non-hardenable by heat treatment and has very poor weld characteristics |  |  |  |
| Very high levels of halide ions, especially the chloride ion can also break down the passive surface film |  |  |  |
| They can be hardened by heat treatment, have poor welding characteristics and are considered magnetic |  |  |  |
| They are suitable only for low concentrations of reducing acids |  |  |  |