Screw Fastener Worksheet

Use the Wikipedia Handout and the Fastenal Catalogue to answer the following questions.

Fill in the missing words:

- The ___________, which has often been described as an inclined plane wrapped around a cylinder, is fundamental to the threaded fastener.

- The tolerance is the ___________between the maximum and minimum permitted limits.

- A screw thread is considered to be a ridge of uniform section in the form of a __________on either the external or internal surface of a cylinder.

- An intentional clearance is created between mating threads known as the_________. This ensures that when both the internal and external threads are manufactured there will be a ____________  ____________between them.

- On an__________  ________, the diameter at the thread crests is the major diameter, and at its thread root is the minor diameter.

- Metric thread pitch is designated as the distance between threads (pitch) in_____________.

- For fasteners, the allowance is generally applied to the__________ _________.

- Internal threads are threads on__________ __________ _______ _________.

- External threads are those on____________ __________ _________.

- The configuration of the thread in an axial plane is the________ _________ __________ ________, and the three parts making the form are the________ ___________ _____ __________.

- The distance-measured parallel to the thread axis, between corresponding points on adjacent threads, is the_________ ___________.

- Unified screw threads are designated in_________ _________ _________. This is the number of complete threads occurring in one inch of threaded length.

- _________ _________ __________have a 30-degree flank angle and are___________ . This is why they are commonly referred to as 60-degree threads.

- On an__________ __________, the diameter at the crests is the minor diameter and at the roots the major diameter.

- Tolerances are specified amounts by which dimensions are permitted to ________ for convenience of manufacturing.

- The __________ _________is the angle between a flank and a perpendicular to the thread axis.

- When flanks have the same angle, the thread is a __________ thread and the flank angle is termed half-angle of thread.

- Pitch diameter is the diameter of a _______ _________that passes through the threads in such a position that the widths of the thread ridges and thread grooves are equal.

- These widths, in a perfect world, would each equal ___________ of the thread pitch.

Information used from Fastenal Reference Guide