PLOTTING INITIALS WITH AUTOCAD: STEP-BY-STEP INSTRUCTIONS FOR INPUTTING ABSOLUTE, RELATIVE, AND POLAR COORDINATES

Points for plotting initials JK with Absolute Coordinates:

2,3

2,2

5,2

5,7

7,2

7,7

7,4

10,7,2

8,5

10,2

| Absolute Coordinate System for Initials JK |  |  |
| --- | --- | --- |
| Absolute Point or Command |  | Description / Referenced from 0,0 |
| 1 | Line Command | Begins line command to input point |
| 2 | 2,3 | 2 in X & 3 in Y axis, first point of letter J |
| 3 | 2,2 | 2 in X & 2 in Y axis, down one unit |
| 4 | 5,2 | 5 in X & 2 in Y, to the right 3 units |
| 5 | 5,7 | 5 in X & 7 in Y, up 5 units |
| 6 | Press Enter | Terminates the line command, Letter J completed |
| 7 | Line Command | Begins command to input point |
| 8 | 7,2 | 7 in X & 2 in Y, begins the letter K |
| 9 | 7,7 | 7 in X & 7 in Y, up 5 units |
| 10 | Press Enter | Terminates the line command |
| 11 | Line Command | Begins command to input point top slant line in K |
| 12 | 7,4 | 7 in X & 4 in Y, beginning point for top slant line in K |
| 13 | 10,7 | 10 in X & 7 in Y end point for top slant in line K |
| 14 | Press Enter | Terminates the line command |
| 15 | 8,5 | 8 in X & 5 in Y, begins lower slant in letter K |
| 16 | 10,2 | 10 in X & 2 in Y, ends lower slant in letter K |
| 17 | Press Enter | Terminates the line command |

Points for plotting initials JK with Relative Coordinates:

2,3

@0,-1

@3,0

@0,5

7,2

@0,5

@0,-3

@3,3

@-2,-2

@2,-3

| Relative Coordinate System for Initials JK |  |  |
| --- | --- | --- |
| Relative Point or Command |  | Description / Referenced from 0,0 |
| 1 | Line Command | Begins line command to input point |
| 2 | 2,3 | 2 in X & 3 in Y axis, first point of letter J |
| 3 | @0,-1 | Down 1 unit in the Y axis |
| 4 | @3,0 | To the right 3 units in X axis |
| 5 | @0,5 | Up 5 units in the Y axis |
| 6 | Press Enter | Terminates the line command, Letter J completed |
| 7 | Line Command | Begins command to input point |
| 8 | 7,2 | 7 in X & 2 in Y, begins the letter K |
| 9 | @0,5 | Up 5 units in the Y axis |
| 10 | @0,-3 | Down 3 units in Y axis, to begin top slant line in K |
| 11 | @3,3 | Up 3 units in X & Y axes, end point of top slant line in K |
| 12 | @-2,-2 | Down -2 in X & Y axes, start point of bottom slant line in K |
| 13 | @2,-3 | 2 in X &-3 in Y axes, end point for bottom slant in line K |
| 14 | Press Enter | Terminates the line command, Initials completed |

Points for plotting initials JK with Polar Coordinates:

2,3

@1<270

@3<0

@%<90

7,2

@5<90

@5<270

@3<45

@3<225

@3.5<310

| Polar Coordinate System for Initials JK |  |  |
| --- | --- | --- |
| Relative Point or Command |  | Description / Referenced from 0,0 |
| 1 | Line Command | Begins line command to input point |
| 2 | 2,3 | 2 in X & 3 in Y axis, first point of letter J |
| 3 | @1<270 | Down 1 unit 270 degrees |
| 4 | @3<0 | To the right 3 units in 0 degrees |
| 5 | @5<90 | Up 5 units 90 degrees |
| 6 | Press Enter | Terminates the line command, Letter J completed |
| 7 | Line Command | Begins command to input point |
| 8 | 7,2 | 7 in X & 2 in Y, begins the letter K |
| 9 | @5<90 | Up 5 units 90 degrees |
| 10 | @3<270 | Down 3 units 270 degrees, to begin top slant line in K |
| 11 | @4<45 | Up 4 units degrees, end point of top slant line in K (estimated) |
| 12 | @3<225 | Down 3 units 270 degrees, start point of bottom slant line in K (estimated) |
| 13 | @3.5<310 | Down 3.5 units 310 degrees, end point for bottom slant in line K (estimated) |
| 14 | Press Enter | Terminates the line command, Initials completed |