Engineering Drawings

The following 2D CAD models will be created:

Drawing 1

Drawing 2

Drawing 2

Create Drawing 1

Step 1 → Select the units type

- Notice the drawing is defined using decimal inches so make sure your drawing units are set to inches showing two decimal places.  Following is an example of a dialog box displayed when entering the UNITS command (or similar):

Step 2 → Define the drawing area

- Use the LIMITS command (or similar) to define the drawing area.  The lower left corner should be 0,0 (zero, zero) and the upper right corner should be 12,12.

Step 3 → Display the drawing area

- Use the ZOOM command with the ALL option (or similar) to display the drawing area.

Step 4 → Create two circles

- Use the CIRCLE command (or similar) to create the first circle

  - Center point = 3,6

  - Diameter = 6

- Use the CIRCLE command (or similar) to create the second circle

  - Center point = 9,6

  - Diameter = 6

Step 5 → Create two lines

- Make sure the QUADRANT object snap is active

  - At the “Command” prompt, type in OSNAP

    - Activate the QUADRANT object snap if necessary

    - Press OK

- Use the LINE command (or similar) to create the first line

  - The first point of the line will be the top quadrant of the left circle

  - The second point of the line will be the top quadrant of the right circle

- Repeat the LINE command to create the bottom line as shown:

Step 6 → Trim the circles

- At the “Command” prompt, select both lines

  - The grips for each line will appear (blue squares, or similar)

- Type in or select the TRIM command (or similar)

- Select both circles where shown (red dots):

Your model should now look like this – a closed loop formed from two arcs and two lines:

Step 7 → Offsetting the arcs and lines

- Use the OFFSET command (or similar)

- Enter the offset distance to be 0.50

- Select any item to offset (either an arc or a line)

- Select anywhere inside the loop defined by the arcs and lines

- Continue offsetting all four objects

Your model should now look like this:

Step 8 → Create a radial pattern

- Use the ARRAY command (or similar)

- Select all objects to pattern

- Select a radial or circular pattern (not linear)

- The center point should be 6,6

- The angle of patterned objects will be 90

- Two objects will be patterned

Your model should look like this:

Step 9 → Trim

- Use the TRIM command (or similar)

  - Select all objects as the cutting edges

  - Select the eight places to trim as shown:

The finished model should look like this:

Create Drawing 2

Step 1 → Select the units type

- Notice the drawing is defined using decimal inches so make sure your drawing units are set to inches showing two decimal places.  Following is an example of a dialog box displayed when entering the UNITS command (or similar):

Step 2 → Define the drawing area

- Use the LIMITS command (or similar) to define the drawing area.  The lower left corner should be 0,0 (zero, zero) and the upper right corner should be 900,900.

Step 3 → Display the drawing area

- Use the ZOOM command with the ALL option (or similar) to display the drawing area

Step 4 → Relocate the User Coordinate System (UCS)

- Use the UCS command (or similar)

- Enter the new origin to be 600,400

  - The UCS icon should now appear near the middle of the drawing window.

Step 5 → Create two lines

- Create the first line:

  - Use the LINE command

    - The start point is 50,100

    - The end point is 50,400	or 	@300<90	 or	@0,300

- Create the second line:

  - Use the LINE command

    - The start point is -50,100

    - The end point is -50,400	or	@300<90	 or	@0,300

Step 6 → Pattern the two lines

- Use the ARRAY command (or similar)

  - Select all objects to pattern

  - Select a radial or circular pattern (not linear)

  - The center point should be 0,0

  - The angle of patterned objects will be 72

  - Two objects will be patterned

Your model should look like this:

Step 7 → Create a circle

- Use the CIRCLE command

  - Center is 0,0

  - Radius is 320

Step 8 → Trim the objects

- Select the TRIM command (or similar)

  - Select all objects as cutting edges

  - Select objects to trim where shown:

Step 9 → Erase some of the objects

- Use the ERASE command (or similar)

  - Only these three objects shown (two lines and one arc) should remain

Step 10 → Add arcs

- Use the FILLET command (or similar)

  - Set the radius to 30

  - Select any two objects to place the arc

  - Repeat until your model looks like this:

Step 11 → Delete the large arc

- Select the large arc, then delete it

- Your model should now look like this:

Step 12 → Pattern all objects

- Use the ARRAY command (or similar)

  - Select all objects to pattern

  - Select a radial or circular pattern (not linear)

  - The center point should be 0,0

  - The angle of patterned objects will be 72

  - Five objects will be patterned

Your model should look like this:

Step 13 → Create three circles

- Use the CIRCLE command

  - Center is 0,0

  - Radius is 320

- Use the CIRCLE command

  - Center is 0,0

  - Radius is 350

- Use the CIRCLE command

  - Center is 0,0

  - Radius is 50

Your model is now finished and should look like this: