# Student Resource 1.1.5

# Automotive Ergonomics

| Path of C. Motion | Control | Dimension (mm) | Force F (N) Moment M (Nm) |  | 2 positions | >2 positions | Continuous adjustment | Precise adjustment | Quick adjustment | Large force application | Tactile feedback | Setting visible | Accidental actuation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Turning Movement | Handwheel |  | D | M |  |  |  |  |  |  |  |  |  |
|  |  | D:160-800 d: 30-40 | 160-200 mm | 2-40 mm 4-60 mm | ◑ | ◑ | ● | ● | ◑ | ● | ○ | ○ | ◑ |
|  | Crank |  | R | M |  |  |  |  |  |  |  |  |  |
|  |  | Hand (Finger) D: <250 (<100) 1: 100 (30) d: 32 (16) | <100 mm 100-250 mm | 0, 6-3 Nm 5-14 Nm | ◑ | ◑ | ● | ◑ | ● | ◑ | ◑ | ◑ | ○ |
|  | Rotary knob |  | D | M |  |  |  |  |  |  |  |  |  |
|  |  | Hand (Finger) D: <25-100 (15-25) H: >20 (>15) | 15-25 mm 25-100 mm | 0, 02-0, 05 Nm 0, 3-0, 7 Nm | ◕ | ◕ | ● | ● | ◑ | ○ | ○ | ○ | ◑ |
|  | Rotary selector switch |  | D | M |  |  |  |  |  |  |  |  |  |
|  |  | 1: 30-70 h: 20 b: 10-25 | 30 mm 30-70 mm | 0, 1-3, 3 Nm 0, 3-0, 6 Nm | ● | ● | ◔ | ◕ | ◔ | ◑ | ● | ● | ◑ |
|  | Thumbwheel |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | b: >8 | 0, 4-5 N |  | ◑ | ◑ | ● | ● | ◑ | ○ | ○ | ○ | ◑ |
|  | Rollball |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | D: 60-120 | 0, 4-5 N |  | ○ | ○ | ● | ● | ◕ | ○ | ○ | ○ | ◑ |

Hand- and foot-operated control devices and their operational characteristics and control functions; ● recommended; ○ not suitable.