Forcible entry vocabulary teachers’ guide

Halligan tool – a tool with an adze, pick, and claw

K tool – used to shear off lock cylinders

A tool – similar to the K tool, but with a pry bar attached

Forcible entry – Method to gain access when normal means of entry cannot be used

PPE – Personal protective equipment

Flat-head axe – an axe with a flat face for striking Halligan tools and other tools

Battering ram – used to force doors and breach walls; takes two to four people to operate

Sledgehammer – used to break down doors or strike other tools

Pry bar – hardened steel tool for forcing doors and windows, removing nails, and separating building materials

Pry axe – in addition to axe blade, has pick and claw

Rabbet tool – small hand-operated hydraulic tool for forcing doors

J tool – used to open double doors with panic bars

Shove knife – used to open older-model doors

Duck-billed lock breakers – tool for opening padlocks

Bam-bam tool – tool for removing lock cylinders

Cylindrical lock – most common fixed lock; pry open

Padlock – most common lock; cut or break shackle

Mortise lock – found in motel rooms, difficult to force; use through-the-lock-techniques

Rim lock – secondary deadbolts, difficult to force; use through-the-lock techniques