Ventilation Vocabulary Teacher’s Guide

Ventilation – The process of removing smoke, heat, and toxic gases from a burning building and replacing them with cooler, cleaner, more oxygen-rich air

Horizontal Ventilation – Utilizes horizontal openings in a structure such as doors and windows

Vertical Ventilation – Involves openings in the roofs or floors

Natural Ventilation – Depends on convection currents, wind, and other natural air movement

Mechanical Ventilation – Uses mechanical means to augment natural ventilation

Negative-Pressure Ventilation – Uses fans called ejectors to exhaust smoke and heat

Positive-Pressure Ventilation – Uses large, powerful fans to force fresh air into a structure

Hydraulic Ventilation – Uses a fog or broken pattern stream to create a pressure differential

Backdraft – An explosion resulting from the introduction of air to a building charged with hot gases and low in oxygen; can be prevented by venting heat without introducing fresh air

Flashover – Sudden ignition of a room filled with hot air in which combustibles are near their ignition point; requires ventilation and cooling