METAR Teacher Guide

Directions: Write this METAR Observation for Los Angeles (listed here below) on the board.

KLAX 150150Z 26010KT 10SM FEW017 BKN200 13/11 A3007 RMK AO2 SLP181 T01280106

Next, decode each METAR item (above) with the reference provided here below.

Conditions at: KLAX (LOS ANGELES, CA, US) observed 0150 UTC 15 January 2006
Temperature: 12.8°C (55°F)
Dewpoint: 10.6°C (51°F) [RH = 86%]
Pressure (altimeter): 30.07 inches Hg (1018.4 mb)
[Sea-level pressure: 1018.1 mb]
Winds: from the W (260 degrees) at 12 MPH (10 knots; 5.2 m/s)
Visibility: 10 or more miles (16+ km)
Ceiling: 20000 feet AGL
Clouds: few clouds at 1700 feet AGL
broken clouds at 20000 feet AGL
Weather: no significant weather observed at this time