Reading METARs Practice Worksheet-KEY

Directions: Practice decoding a METAR using the following Georgia airports for February.

*Teacher note: Students may not know the code for mist, rain ended, tower visibility, and ceiling.

Athens: Athens/Ben Epps 
KAHN 141604Z 17003KT 4SM -RA BR BKN023 OVC046 04/02 A3010 RMK AO2 RAB01 P0000

ICAO station identifier: KAHN on the 14th day of the month at 16:04z (11:04 a.m. EST); Wind S (170 degrees) at 03 knots (KT); Visibility: 4 miles; Light Rain; Broken clouds present at 2,300 feet, overcast clouds present at 4,600 feet; 4°Celcius, dew point 2° Celsius; Pressure (altimeter): 30.10 inches Hg; automated observation with precipitation discriminator (rain/snow); 00.00 inches of rain in the last hour

Albany: Southwest Georgia Regional
KABY 141557Z 13007KT 8SM BKN034 OVC043 12/06 A3009 RMK AO2

ICAO Station identifier: KABY on the 14th day of the month at 15:57z (10:57 a.m. EST); Wind SE (130 degrees) at 07 knots (KT); Visibility 8 miles; Broken clouds present at 3,400 feet and overcast clouds present at 4,300 feet; 12° Celsius, dew point 6° Celsius; Pressure (altimeter): 30.09 inches Hg; automated observation with precipitation discriminator (rain/snow).

Atlanta: Hartsfield-Jackson Atlanta International
KATL 141552Z 15004KT 6SM HZ FEW008 BKN025 OVC030 05/02 A3007 RMK AO2 RAE1455 SLP186 P0000 T00500022

ICAO Station identifier: KATL on the 14th day of the month at 15:52z (10:52 a.m. EST); Wind S-SE (150 degrees) at 04 knots (KT); Visibility 6 miles; Few clouds 800ft., Broken clouds present 2,500 feet, overcast clouds present at 3,000 feet; 5° Celsius, dew point 2° Celsius; Pressure (altimeter): 30.07 inches Hg; automated observation with precipitation discriminator (rain/snow). Rain ended 14:55 (9:55 a.m. EST); Sea Level Pressure 1018.6 mb; 00.00 inches liquid precipitation in last hour. Temperature: 5.0° Celsius, dew point 2.2° Celsius.

Savannah: Savannah/Hilton Head International KSAV
KSAV 141553Z 14004KT 3SM BR FEW005 SCT040 OVC085 09/08 A3015 RMK AO2 TWR VIS 4 RAE52 SLP208 P0002 T00940078

ICAO Station identifier: KSAV on the 14th day of the month at 15:53z (10:53 a.m. EST); Wind SE (140 degrees) at 04 knots (KT); Visibility 3 miles;Mist,  Few clouds 500ft., Scattered clouds present 4,000 feet, overcast clouds present at 8,500 feet; 9° Celsius, dew point 8° Celsius; Pressure (altimeter): 30.15 inches Hg; automated observation with precipitation discriminator (rain/snow), Tower visibility . Rain ended 00:52z (12:52 a.m. EST); Sea Level Pressure 1020.8 mb; 00.02 inches liquid precipitation in last hour. Temperature: 9.4° Celsius, dew point: 7.8° Celsius.

Rome: Richard B. Russel
KRMG 141610Z AUTO 12004KT 7SM BKN007 OVC011 04/03 A3006 RMK AO2 CIG 005V010
ICAO Station identifier: KRMG on the 14th day of the month at 16:10z (11:10 a.m. EST); Site attended (manual observation); Wind E-SE (120 degrees) at 04 knots (KT); Visibility 7 miles;  Broken clouds present at 700 feet, overcast clouds present at 1,100 feet; 4° Celsius, dew point 3° Celsius; Pressure (altimeter): 30.06 inches Hg; automated observation with precipitation discriminator (rain/snow), Ceiling  varying 500 and 1,000 feet.