- International code used to report surface weather observations:
(a) LIDAR  (b) METAR  (c) PIREP  (d) SONAR

2.	Cloud bases are reported in ________.

a.  Hundreds of feet above sea level (MSL)

b.  Hundreds of feet above the ground level (AGL)

c.  Thousands of feet above sea level (MSL)

d.  Thousands of feet above ground level (AGL)

3.	Weather reported as being in the vicinity (VC) is between _____ statute miles from the reporting station.

a. 	5 and 10	              b.      10 and 15

b.	15 and 20)                      c.      All of the above

5. 	Given the following TAF, what is the forecast wind direction and velocity for 5 a.m. local time (eight hours behind UTC)?

KHQM 231740Z 2318/2418 15004KT P6SM SCT010 OVC020

FM232200 12006KT P6SM SCT010

FM240500 07004KT 5SM BR SCT006

FM241200 08009KT 3SM BR BKN005

- Wind from 150 degrees at four knots

- Wind from 120 degrees at six

- Wind from 070 degrees at four knots

- Wind from 007 degrees at four knots

6.	"VCSH" in a METAR report means:

a. Volcanic ash.

b. Showers in the vicinity.

c. Showers very close to the observation point.

d. Virga and showers.

7.	During your preflight planning, you see the TAF below for Gunnison, Colorado (KGUC). What does it mean? 

KGUC 241120Z 2412/2512 NIL

a.	There is no significant weather to report

b.	The forecast has not changed since the previous TAF

8.	In order to qualify as a ceiling, the sky condition has to be reported as at least _____.

a.   Few (FEW)      b.   Scattered (SCT)    c.  Broken (BKN)   d. Any of the above

9.  	What is the difference between SKC and CLR in METAR reports?

a. SKC is reported at manual stations when the sky is clear, and CLR is reported at automated stations when the sky is clear.

b. CLR is reported at manual stations when the sky is clear, and SKC is reported at automated stations when the sky is clear below 12,000 feet.

c. SKC is reported at manual stations when the sky is clear, and CLR is reported at automated stations when the sky is clear below 12,000 feet.

d. SKC and CLR are interchangeable.

10.	In a METAR report, "FEW025" describes ___________.

a.  scattered layer covering 1/8 to 2/8 of the sky, height unknown.

b.  sky partially obscured, obscuration covering 1/8 to 2/8 of the sky.

c.  scattered layer covering 1/8 to 2/8 of the sky, height greater than 12,000 feet (reported only by AWOS stations).

d.  scattered layer at 2500 feet that covers 1/8 to 2/8 of the sky