Screen Printing Test Post-TEST (70 points)

Name __________________________________

Period _____

Date _______

Multiple Choice – circle the most correct answer (2 points each)

- Which is not a method to create artwork for t-shirts?

  - original artwork using a black sharpie or dark pen

  - computer clip art

  - google image (large size is best)

  - photograph with contrast improved in Photoshop

  - light blue pencil

- What tools are needed to center the image on the screen with the edge of the platen?

  - t-square

  - carpenter’s square

  - ruler

  - “a” and “c”

  - “a” and “b”

- What terms are used interchangeably with screen stencils in the printing industry?

  - serigraphy

  - screen printing

  - silkscreen printing

  - all of the above

- What mesh count (openings per inch) is best for printing white ink on black shirts?

  - 110

  - 280

  - 305

  - any mesh count will work

- After scraping the screen of excess ink, what do you use to clean ink from the screen after you take it off the press?

  - water in the sink

  - emulsion remover

  - screen wash

  - none of the above

- Emulsion is applied to the screen with a __________.

  - scoop coater

  - small nylon brush

  - large camelhair brush

  - trowel

- When printing dark ink on a white shirt, the squeegee should be held at the following angle to get the best results:

  - approximately 10 degrees

  - approximately 90 degrees

  - approximately 45 degrees

  - doesn’t matter

- What type of ink is primarily used for t-shirt printing in our shop?

  - acrylic paint

  - water-based ink

  - plastisol ink

  - non-heat set ink

- What are the two sides of the screen called?

  - long side, short side

  - wrong side, right side

  - print side, shirt side

  - print side, ink side

- When using a low pressure spray to wash out the unexposed emulsion after the screen is “burned”, the design should be completely washed out in what amount of time?

  - 10 seconds

  - about one minute

  - about 10 minutes

  - only the exposed emulsion is washed out

- Screens coated with emulsion are “burned” for what amount of time?

  - 1 minute

  - 2 minutes

  - 3 minutes

  - exposure depends on previous test results

- The ideal surface temperature to cure the ink as it goes through the dryer is:

  - 200-250 degrees Fahrenheit

  - exactly  176 degrees Fahrenheit

  - 300-350 degrees Fahrenheit

  - Until it starts to bubble

- The tool used to measure the ink temperature is called:

  - a plastinonimetric indicator

  - a non-contact thermometer

  - an outdoor thermometer

  - you don’t really need to measure the temperature

- The type of frame that allows the mesh to be tightened while it is still on the frame is called:

  - a plastic frame

  - a metal roller frame

  - a wood frame

  - none of the above

- What kind of press do you use to print a t-shirt with  2,3,4, 5 or 6 colors?

  - standard press

  - offset press

  - rotary press

  - full court press

- When printing a two-color (red and white ink) heart design, you need:

  - one screen

  - two screen

  - three screens

  - four screens

- What are the reasons for removing the old mesh from a wooden frame and replacing it with new mesh?

  - blocked openings in the mesh

  - loose screen

  - small tears

  - all of the above

- Select the factors that best determines the quantity of ink placed on the t-shirt.

  - squeegee angle only

  - squeegee angle, pressure

  - squeegee angle, pressure, speed, amount of ink, mesh count

  - squeegee sharpness

- Small holes in the screen emulsion that are caused by dust/dirt on the exposure unit glass, and are fixed with blockout and a small paint brush, are called:

  - dust holes

  - pin holes

  - brush holes

  - screen holes

- Blockout is applied with a scraper to what side of the screen?

  - ink side

  - off side

  - on side

  - print side

True or False- Circle T or F (2 points each)

T     F     21.  The ink used to print t-shirts will not air dry on the screen.

T     F     22.  When “burning” a screen, the translucent positive is taped to the ink side with the darker image facing down.

T     F     23.  If you get blockout on your image area you will need to burn a new screen.

T     F     24.  We use pure silk fabric mesh to make the screens in our shop.

T     F     25.  During the Sun Dynasty in Japan, artists used human hair to hold loose parts of the stencil.

Excluding something that is worn on the body, name 3 everyday items that are screen printed. (2 points each)

26.

27.

28.

Match the word or phrase with the letter and best answer to the right.  (2 points each)

_____  29.  Halftone

_____  30.  Table adhesive

_____  31.  Separations

_____  32.  Moire

_____  33.  Flash unit

_____  34.  Wholesale cost

_____  35.  Retail price

- Dots that add shading and depth to a design and dictate how much of each color goes on the t-shirt material.  Photographs on t-shirts use these.

- This is how much the shop owner pays the warehouse or manufacturer to buy goods later sold to the consumer.

- Unwanted lines or patterns caused by the interaction of halftones and screen mesh

- In a multicolored design, the printer must accurately position these sheets of film or translucent paper on each screen to be burned so that all the colors will line up (registration) correctly when the shirt is printed.

- This is used to keep the t-shirt from moving on the platen during printing.

- This is how much the shop owner charges the customer.

- This is used on light colored inks to heat set each layer of ink to make it more opaque.