Screen Printing Notes

Advantages of Screen Printing

- Low Capital Investment

- Ease of operation/simple to do

- Ability to print glitter, puff ink and specialty inks

- Versatility of substrates on which to print

- Strength and durability of ink because of ink film thickness

Disadvantages of Screen Printing

- Poor ink mileage

- Inability to print fine detail compared to other processes

- Extended drying time with certain types of ink

- Slow production rate

Printing Production

IMAGE PREPARATION

- Thumbnail sketches

- Rough layout

- Comprehensive layout

- Final printout from Illustrator

- Print vellum on laser printer and spray with clear plastic spray

SCREEN PREPARATION

- ABRADE or ROUGHEN screen if brand new fabric on the frame

- If screen has already been used, DEGREASE screen

- Let it Dry

- Screen gets Coated with Emulsion (your instructor will do this)

- Let it Dry

- EXPOSE screen with vellum/positive

- WASH OUT IMAGE (sink) with gentle spray

- BLOW OUT AROUND IMAGE AREA (air pressure)

- Let it Dry

- TAPE OUT SCREEN

- BLOCKOUT pinholes if necessary

Printing Terms: Flood Stroke, print stroke, platen, plastisol ink, squeegee, adhesive spray, double image, screen wash, blockout, registration/register, ER-35.