# Temperature Outline

Measurement of balance of heat loss and heat produced

Abbreviation T

Homeostasis – constant state of fluid balance

Body reacts to chemicals and influences temp

Sites to measure T

Rectal - rectum

Mouth - oral

Axillary - armpit

Aural – ear

Temporal – forehead

Factors that affect body temp

Individual people differ – metabolic rates

Time of day

Body Sites

Activities

Causes of increase – illness, infection, exercise, excitement, 				 environment

Cause of decrease – starvation of fasting, ↓ muscle activity, 				mouth breathing, and exposure to cold & certain disease

Methods to Measure Temp

Oral

Most comfortable and common

Questions pt about eat drink or smoke prior to temp

Leave in place 3-5 minutes if using merciless thermometer

Digital – leave until beeps usually  one minute

Electronic – records within 2 – 4 seconds

Tympanic – record aural readings, placed in the ear canal uses inferred 			reading of the tympanic membrane.   Must be used 					correctly for accuracy

Temporal – measure the temporal artery

Terminology related to temp

- Hypothermia – low body temp  ↓ 95°

- Hyperthermia – high body temp  104° F

- Fever – an elevated (↑) temp  usually 101°F

- Pyrexia – another term for fever

How to read a glass thermometer

The long line represents a whole number ex 98°

The short line represents .2 ° (2 tenths) of a degree

Normal Ranges:

Oral = 	98.6° F      (+ or - 1° )   37° C

Rectal = 	99.6° F      (+ or - 1° )   37.6° C

Axillary =	97.6° F      (+ or - 1° )   36.4° C

Guidelines for Obtaining a Oral Temp

- Standard Precautions – wipe with alcohol or facility guideline before and after use; cover tip/probe; check glass thermometer prior to use, make sure the line is below 96° careful when shaking down not to hit objects close by.  Use cool water when rinsing to prevent from breaking glass and destroying contents inside of the thermometer

- Record and Report

- Supplies for Temp

Oral thermometer, plastic sheath, holder of with disinfectant, 	tissues or dry cotton balls, watch with second hand, soapy cotton 	balls, gloves, paper and pen.