# Duration of Flow

Read the following scenarios and perform the mathematics required to determine the duration of flow:

Cylinder Constants:

D=0.16

E=0.28

EQUATION:

[(Gauge pressure in PSI – 200PSI) x Cylinder Constant]/flow rate in liters per minute

Example:

Patient A is receiving 5 LPM of oxygen from an E tank that has 1000PSI.

[(1000-200) x 0.28] / 5 = 44.8 minutes

The patient is receiving the following:

- 10 LPM of oxygen from a D tank that has 2000psi

- 8 LPM of oxygen from an E tank that has 1800 psi

- 2 LPM of oxygen from an E tank that has 500 psi

- 15 LPM of oxygen from an E tank that has 1200psi

- 6 LPM of oxygen from a D tank that has 700psi

- 3 LPM of oxygen from a D tank that has 300psi

- 12 LPM of oxygen from a E tank that has 1600 psi

- 1 LPM of oxygen from a D tank with 2000 psi

- 15 LPM of oxygen from a E tank with 2000 psi

- 8 LPM of oxygen from a D tank with 1000 psi