NIOSH Pocket Guide to Hazardous Chemicals Flammable and Combustible Liquid Classes When possible, the flammability/combustibility of a substance was determined and listed after the specific gravity. The following OSHA criteria (29 CFR 1910.106) were used to classify flammable or combustible liquids:. Class IA flammable liquid; Fl.P. below 73°F and BP below 100°F.. Class IB flammable liquid; Fl.P. below 73°F and BP at or above 100°F.. Class IC flammable liquid; Fl.P. at or above 73°F and below 100°F.. Class II combustible liquid; Fl.P. at or above 100°F and below 140°F.. Class IIIA combustible liquid; Fl.P. at or above 140°F and below 200°F.. Class IIIB combustible liquid; Fl.P. at or above 200°F.. First Aid Procedures. Code; Definition. Eye: Irrigate immediately; If this chemical contacts the eyes, immediately wash the eyes with large amounts of water, occasionally lifting the lower and upper lids. Get medical attention immediately. Contact lenses should not be worn when working with this chemical.. Eye: Irrigate promptly; If this chemical contacts the eyes, promptly wash the eyes with large amounts of water, occasionally lifting the lower and upper lids. Get medical attention If any discomfort continues. Contact lenses should not be worn when working with this chemical.. Eye: Frostbite; If eye tissue is frozen, seek medical attention immediately; If tissue is not frozen, immediately and thoroughly flush the eyes with large amounts of water for at least 15 minutes, occasionally lifting the lower and upper eyelids. If irritation, pain, swelling, lacrimation, or photophobia persist, get medical attention as soon as possible.. Eye: Medical attention; Self-explanatory. Skin: Blot/brush away; If irritation occurs, gently blot or brush away excess.. Skin: Dust off solid; water flush; If this solid chemical contacts the skin, dust it off immediately and Then flush the contaminated skin with water. If this chemical or liquids containing this chemical penetrate the clothing, promptly remove the clothing and flush the skin with water. Get medical attention immediately.. Skin: Frostbite; If frostbite has occurred, seek medical attention immediately; do NOT rub the affected areas or flush them with water. In order to prevent further tissue damage, do NOT attempt to remove frozen clothing from frostbitten areas. If frostbite has NOT occurred, immediately and thoroughly wash contaminated skin with soap and water.. Skin: Molten flush immediately/solid-liquid soap wash immediately; If this molten chemical contacts the skin, immediately flush the skin with large amounts of water. Get medical attention immediately. If this chemical (or liquids containing this chemical) contacts the skin, promptly wash the contaminated skin with soap and water. If this chemical or liquids containing this chemical penetrate the clothing, immediately remove the clothing and wash the skin with soap and water. If irritation persists after washing, get medical attention.. Skin: Soap flush immediately; If this chemical contacts the skin, immediately flush the contaminated skin with soap and water. If this chemical penetrates the clothing, immediately remove the clothing and flush the skin with water. If irritation persists after washing, get medical attention.. Skin: Soap flush promptly; If this chemical contacts the skin, promptly flush the contaminated skin with soap and water. If this chemical penetrates the clothing, promptly remove the clothing and flush the skin with water. If irritation persists after washing, get medical attention.. Skin: Soap promptly/molten flush immediately; If this solid chemical or a liquid containing this chemical contacts the skin, promptly wash the contaminated skin with soap and water. If irritation persists after washing, get medical attention. If this molten chemical contacts the skin or nonimpervious clothing, immediately flush the affected area with large amounts of water to remove heat. Get medical attention immediately.. Skin: Soap wash; If this chemical contacts the skin, wash the contaminated skin with soap and water.. Skin: Soap wash immediately; If this chemical contacts the skin, immediately wash the contaminated skin with soap and water. If this chemical penetrates the clothing, immediately remove the clothing, wash the skin with soap and water, and get medical attention promptly.. Skin: Soap wash promptly; If this chemical contacts the skin, promptly wash the contaminated skin with soap and water. If this chemical penetrates the clothing, promptly remove the clothing and wash the skin with soap and water. Get medical attention promptly.. Skin: Water flush; If this chemical contacts the skin, flush the contaminated skin with water. WHERE there is evidence of skin irritation, get medical attention.. Skin: Water flush immediately; If this chemical contacts the skin, immediately flush the contaminated skin with water. If this chemical penetrates the clothing, immediately remove the clothing and flush the skin with water. Get medical attention promptly.. Skin: Water flush promptly; If this chemical contacts the skin, flush the contaminated skin with water promptly. If this chemical penetrates the clothing, immediately remove the clothing and flush the skin with water promptly. If irritation persists after washing, get medical attention.. Skin: Water wash; If this chemical contacts the skin, wash the contaminated skin with water.. Skin: Water wash immediately; If this chemical contacts the skin, immediately wash the contaminated skin with water. If this chemical penetrates the clothing, immediately remove the clothing and wash the skin with water. If symptoms occur after washing, get medical attention immediately.. Skin: Water wash promptly; If this chemical contacts the skin, promptly wash the contaminated skin with water. If this chemical penetrates the clothing, promptly remove the clothing and wash the skin with water. If irritation persists after washing, get medical attention.. Breath: Respiratory support; If a person breathes large amounts of this chemical, move the exposed person to fresh air at once. If breathing has stopped, perform mouth-to-mouth resuscitation. Keep the affected person warm and at rest. Get medical attention as soon as possible.. Breath: Fresh air; If a person breathes large amounts of this chemical, move the exposed person to fresh air at once. Other measures are usually unnecessary.. Breath: Fresh air, 100% O2; If a person breathes large amounts of this chemical, move the exposed person to fresh air at once. If breathing has stopped, perform artificial respiration. When breathing is difficult, properly trained personnel may assist the affected person by administering 100% oxygen. Keep the affected person warm and at rest. Get medical attention as soon as possible.. Swallow: Medical attention immediately; If this chemical has been swallowed, get medical attention immediately.. Symbols, Code Components, and Codes Used for Respirator Selection. Symbols:. Item; Definition. ¥; At concentrations above the NIOSH REL, or WHERE there is no REL, at any detectable concentration. §; Emergency or planned entry into unknown concentrations or IDLH conditions. *; Substance reported to cause eye irritation or damage; may require eye protection. £; Substance causes eye irritation or damage; eye protection needed. ^; If not present as a fume. ¿; Only nonoxidizable sorbents allowed (not charcoal). †; End of service life indicator (ESLI) required. APF; Assigned protection factor. TBAL; To be added later. Code Component:. Item; Definition. CCR; Chemical Cartridge respirator. D; Dust respirator (If an independent code); or a dust filter. F; Full facepiece. Fu; Fume filter. GMF; Air-purifying, full-facepiece respirator (gas mask) with a chin-style, front- or back-mounted canister. M; Mist filter. PAPR; Powered, air-purifying respirator. SA; Supplied-air respirator. SCBA; Self-contained breathing apparatus. AG; Acid gas cartridge or canister. CF; Continuous flow mode. HiE; Air-purifying respirator with a high-efficiency particulate filter (If an independent code); or a high-efficiency particulate filter. HiEF; Air-purifying, full-facepiece respirator with a high-efficiency particulate filter. OV; Organic vapor cartridge or canister. PD,PP; Pressure-demand or other positive-pressure mode. S; Chemical cartridge or canister providing protection against the compound of concern. T; Tight-fitting facepiece. XS; Except single-use respirator. XSQ; Except single-use and quarter-mask respirator. Code:. Item; Definition. CCRFAGHiE (APF=50); Any chemical cartridge respirator with a full facepiece and acid gas cartridge(s) in combination with a high-efficiency particulate filter. CCRFOV (APF = 50); Any chemical cartridge respirator with a full facepiece and organic vapor cartridge(s). CCRFOVDMFu (APF = 50); Any chemical cartridge respirator with a full facepiece and organic vapor cartridge(s) in combination with a dust, mist, and fume filter. CCRFOVHiE (APF = 50); Any chemical cartridge respirator with a full facepiece and organic vapor cartridge(s) in combination with a high-efficiency particulate filter. CCRFS (APF = 50); Any chemical cartridge respirator with a full facepiece and cartridge(s) providing protection against the compound of concern. CCRFSHiE (APF = 50); Any chemical cartridge respirator with a full facepiece and cartridge(s) providing protection aginst the compound of concern and having a high-efficiency particulate filter. CCROV (APF = 10); Any chemical cartridge respirator with organic vapor cartridge(s). CCROVAG (APF = 10); Any chemical cartridge respirator with organic vapor and acid gas cartridge(s). CCROVDM (APF = 10); Any chemical cartridge respirator with organic vapor cartridge(s) in combination with a dust and mist filter. CCROVDMFu (APF = 10); Any chemical cartridge respirator with organic vapor cartridge(s) in combination with a dust, mist, and fume filter. CCROVHiE (APF = 10); Any chemical cartridge respirator with organic vapor cartridge(s) in combination with a high-efficiency particulate filter. CCRS (APF = 10); Any chemical cartridge respirator with organic vapor cartridge(s) providing protection against the compound of concern. D (APF = 5); Any dust respirator. DM (APF = 5); Any dust and mist respirator. DMF (APF = 10); Any dust and mist respirator with a full facepiece. DMFu (APF = 10); Any dust, mist and fume respirator. DMXSQ (APF = 10); Any dust and mist respirator except single-use and quarter-mask respirators. DXSQ (APF = 10); Any dust respirator except single-use and quarter-mask respirators. GMFAG (APF = 50); Any air-purifying, full-facepiece respirator (gas mask) with a chin-style, front- or back-mounted acid gas canister. CMFAGHiE (APF = 50); Any air-purifying, full-facepiece respirator (gas mask) with a chin-style, front- or back-mounted acid gas canister having a high-efficiency particulate filter. GMFOV (APF = 50); Any air-purifying, full-facepiece respirator (gas mask) with a chin-style, front- or back-mounted organic vapor canister. GMFOVAG (APF = 50); Any air-purifying, full-facepiece respirator (gas mask) with a chin-style, front- or back-mounted organic vapor and acid gas canister. GMFOVAGHiE (APF = 50); Any air-purifying, full-facepiece respirator (gas mask) with a chin-style, front- or back-mounted organic vapor and acid gas canister having a high-efficiency particulate filter. GMFOVDMFu (APF = 50); Any air-purifying, full-facepiece respirator (gas mask) with a chin-style, front- or back-mounted organic vapor canister in combination with a dust, mist, and fume filter. GMFOVHiE (APF = 50); Any air-purifying, full-facepiece respirator (gas mask) with a chin-style, front- or back-mounted organic vapor canister having a high-efficiency particulate filter. GMFS (APF = 50); Any air-purifying, full-facepiece respirator (gas mask) with a chin-style, front- or back-mounted canister providing protection against the compound of concern. GMFSHiE (APF = 50); Any air-purifying, full-facepiece respirator (gas mask) with a chin-style, front- or back-mounted canister providing protection against the compound of concern and having a high-efficiency particulate filter. HiE (APF = 10); Any air-purifying respirator with a high-efficiency particulate filter. HiEF (APF = 50); Any air-purifying, full-facepiece respirator with a high-efficiency particulate filter. PAPRAG (APF = 25); Any powered, air-purifying respirator with acid gas cartridge(s). PAPRAGHiE (APF = 25); Any powered, air-purifying respirator with acid gas cartridge(s) in combination with a high-efficiency particulate filter. PAPRD (APF = 25); Any powered, air-purifying respirator with a dust filter. PAPRDM (APF = 25); Any powered, air-purifying respirator with a dust and mist filter. PAPRDMFu (APF = 25); Any powered, air-purifying respirator with a dust, mist, and fume filter. PAPRHiE (APF = 25); Any powered, air-purifying respirator with a high-efficiency particulate filter. PAPROV (APF = 25); Any powered, air-purifying respirator with organic vapor cartidge(s). PAPROVAG (APF = 25); Any powered, air-purifying respirator with organic vapor and acid gas cartidge(s). PAPROVDMFu (APF = 25); Any powered, air-purifying respirator with organic vapor cartidge(s) in combination with a dust, mist, and fume filter. PAPRS (APF = 25); Any powered, air-purifying respirator with cartidge(s) providing protection against the compound of concern. PAPRTHiE (APF = 50); Any powered, air-purifying respirator with a tight-fitting facepiece and a high-efficiency particulate filter. PAPRTOV (APF = 50); Any powered, air-purifying respirator with a tight-fitting facepiece and organic vapor cartridge(s). PAPRTOVHiE (APF = 50); Any powered, air-purifying respirator with a tight-fitting facepiece and organic vapor cartridge(s) in combination with a high-efficiency particulate filter. PAPRTS (APF = 50); Any powered, air-purifying respirator with a tight-fitting facepiece and cartridge(s) providing protection against the compound of concern. SA (APF = 10); Any supplied-air respirator. SA:CF (APF = 25); Any supplied-air respirator operated in the continuous-flow mode. SAF (APF = 50); Any supplied-air respirator with a full facepiece. SAF:PD,PP (APF = 2,000); Any supplied-air respirator that has a full facepiece and is operated in a pressure-demand or other positive-pressure mode. SAF:PD,PP:ASCBA (APF = 10,000); Any supplied-air respirator that has a full facepiece and is operated in a pressure-demand or other positive pressure mode in combination with an auxiliary self-contained breathing apparatus operated in pressure-demand or other positive-pressure mode. SA:PD,PP (APF = 1,000); Any supplied-air respirator operated in a pressure-demand or other positive pressure mode. SAT:CF (APF = 50); Any supplied-air respirator that has a tight-fitting facepiece and is operated in a continuous-flow mode. SCBAE; Any appropriate escape-type, self-contained breathing apparatus. SCBAF (APF = 50); Any self-contained breathing apparatus with a facepiece. SCBAF:PD,PP (APF = 10,000); Any self-contained breathing apparatus that has a full facepiece and is operated in a pressure-demand or other positive-pressure mode. Symptoms of Exposure and Target Organs. Code; Definition. abdom; Abdominal. abnor; Abnormal/Abnormalities. abs; Skin absorption. album; Albuminuria. anes; Anesthesia. anor; Anorexia. anos; Anosmia (loss of the sense of smell). anxi; Anxiety. arrhy; Arrhythmias. aspir; Aspiration. asphy; Asphyxia. BP; Blood pressure. breath; Breath/breathing. bron; Bronchitis. BUN; Blood urea nitrogen. [carc]; Potential occupational carcinogen. card; Cardiac. chol; Cholinesterase. cirr; Cirrhosis. CNS; Central nervous system. conc; Concentration. con; Skin and/or eye contact. conf; Confusion. conj; Conjunctivitis. constip; Constipation. convuls; Convulsions. corn; Corneal. CVS; Cardiovascular system. cyan; Cyanosis. decr; Decreased. depres; Depressed/Depression. derm; Dermatitis. diarr; Diarrhea. dist; Disturbance. dizz; Dizziness. drow; Drowsiness. dysp; Dyspnea (breathing difficulty). emphy; Emphysema. eosin; Eosinophilia. epilep; Epileptiform. epis; Epistaxis (nosebleed). equi; Equilibrium. eryt; Erythema (skin redness). euph; Euphoria. fail; Failure. fasc; Fasciculation. FEV; Forced expiratory volume. fib; Fibrosis. ftg; Fatigue. func; Function. GI; Gastrointestinal. halu; Hallucinations. head; Headache. hema; Hematuria (blood in the urine). hemato; Hematopoietic. hemorr; Hemorrhage. hyperpig; Hyperpigmentation. hypox; Hypoxemia (reduced oxygen in the blood). inco; Incoordination. incr; Increased. inebri; Inebriation. inflamm; Inflammation. ing; Ingestion. inh; Inhalation. inj; Injury. insom; Insomnia. irreg; Irregular/Irregularities. irrit; Irritation. irrity; Irritability. jaun; Jaundice. kera; Keratitis (inflammation of the cornea). lac; Lacrimation (discharge of tears). lar; Laryngeal. lass; Lassitude (weakness, exhaustion). leucyt; Leukocytosis (increased blood leukocytes). leupen; Leukopenia (reduced blood leukocytes). liq; Liquid. local; Localized. low-wgt; Weight loss. mal; Malaise (vague feeling of discomfort). malnut; Malnutrition. methemo; Methemoglobinemia. muc memb; Mucous membrane. musc; Muscle. narco; Narcosis. nau; Nausea. nec; Necrosis. neph; Nephritis. numb; Numb/numbness. opac; Opacity. palp; Palpitations. para; Paralysis. pares; Paresthesia. perf; Perforation. peri neur; Peripheral neuropathy. periorb; Periorbital (situated around the eye). phar; Pharyngeal. photo; Photophobia (abnormal visual intolerance to light). pneu; Pneumonitis. polyneur; Polyneuropathy. prot; Proteinuria. pulm; Pulmonary. RBC; Red blood cell. repro; Reproductive. resp; Respiratory/respiration. restless; Restlessness. retster; Retrosternal (occurring behind the sternum). rhin; Rhinorrhea (discharge of thin nasal mucus). salv; Salivation. sens; Sensitization. short; Shortness. sneez; Sneezing. sol; Solid. soln; Solution. subs; Substernal (occurring beneath the sternum). sweat; Sweating. swell; Swelling. sys; System. tacar; Tachycardia. tend; Tenderness. terato; Teratogenic. throb; Throbbing. tight; Tightness. twitch; Twitching. uncon; Unconsciousness. vap; Vapor. vesic; Vesiculation. vis; Visual. vomit; Vomiting. weak; Weak/weakness. wheez; Wheezing.

# OSHA/EPA Occupational Chemical Database
Acronyms & Notes