μμ

PRACTICE PIPETTING

Exercise One

DIRECTIONS:  Follow the directions below for pipetting practice.

- Label four tubes (13mm x 100mm) with 1, 2, 3, & 4.

- Select & use the smallest pipette possible for accuracy.

- Allow the solution to flow down the inside of each tube.

- Pipette & fill tubes according to the chart below.

[Solution I – Yellow Dye; Solution II – Blue Dye; Solution – Red Dye]

Tubes:     Water      Solution I     Solution II     Solution III         = Total Volume

# 1           3.8 ML            0.5 ML            1.2 ML            1.5 ML                   7.0 ML

# 2           2.5 ML            1.7 ML            2.5 ML            0.3 ML                   7.0 ML

# 3           4.5 ML            1.9 ML           0.5 ML             0.1 ML                   7.0 ML

# 4           2.0 ML            2.6 ML           1.0 ML             1.4 ML                   7.0 ML

- After mixing compare your tubes with the standard tubes. Repeat as needed.

Exercise Two

- Label two reaction tubes as 1 & 2.

- Add the smallest volume first & let it run down the inside of tube.

- Use a P-10 or P-20 micropipette & the same solutions as the previous exercise.

Tubes:    Water     Solution I      Solution II    Solution III            = Total Volume

#1           5.0 μ           4.5 μ                3.0 μ                2.5  μ                      15 μ

#2           5.0 μ          2.5 μ                 3.0 μ                4.5  μ                      15 μ

4.  Spin tubes for a few seconds to pool the solutions & compare with standards.