Fire Rescue Unit Exam KEY

Multiple choice: Please select the best answer choice for each question.

1. What should firefighters do as soon as they arrive at the scene of a building fire?

- immediately prepare for a hose attack

- promptly enter the building and begin a rescue search

- carefully observe the building

- rely solely on size-up information provided by the first arriving officer

2. When should firefighters identify alternate escape routes from a burning building?

- during pre-incident planning

- before entering the building

- upon discovering routes inside the building

- after determining the need

3. What should firefighters do with information obtained from occupants who have escaped the building?

- relay it to the incident commander

- transmit it over the radio to crews entering the building

- forward it to the victims’ families and friend

- disregard it, and allow firefighters to confirm the actual status

4. What are the two objectives of a building search?

- finding victims and discovering arson clues

- searching for life and searching for fire extension

- saving lives and removing bodies

- rescuing victims and preventing fire spread

5. Who should conduct the secondary search of a building?

- Firefighters who conducted the primary search

- arson investigators who are also trained in first aid

- special search teams assigned to conduct secondary searches at every fire

- personnel who were not involved in the primary search

6. How should rescuers search the fire floor?

- begin at the entrance and send one team in a clockwise direction and the other in a clockwise direction

- begin at the entrance and search in one direction only

- begin as close to the fire as possible and search back toward the entrance

- begin as far away from the fire as possible and search toward the fire

7. How should firefighters search each room of a building?

- search the perimeter first, and then search the middle of the room

- search the middle of the room, moving in a circular pattern to the perimeter of the room

- search from the front of the room, zigzagging to the back of the room

- search each room differently, giving the most attention to bedrooms and living areas

8. Which of the following is not a basic rule that firefighters should follow when searching a building?

- Turn left to leave the room if you turned left to enter the room.

- Turn right to leave the room if you turned left to enter the room

- Exit each room through the same doorway that you entered

- Turn the opposite direction used to enter the room when removing a victim.

9. What is the best method for a team to search small rooms?

- both team members should move together to search each room

- team members should remain at the door while the other member searches to room

- one team member should remain at the door while the other member searches to room

- one team member should search under beds, in closets, and in all enclosed places while the other member searches open areas

10. Which of the following actions is recommended for a firefighter who becomes disoriented?

- shouting for help periodically

- finding a place of relative safety and waiting for help

- walking toward a lighted area

- following the hoseline in the same direction as the female couple

True or False

__False___11. Firefighters do not need to conduct a typical search for victims if the fire in a building is small and easily extinguished

__False___12. Firefighters do not need to search a burning apartment building if the occupants who have escape can confirm that it is empty.

_False___13. Rescuers should work separately during a primary search.

_False____14. Primary search personnel should not carry tools with them when they enter a building to conduct a search.

_True____15. During search operations, negative information is just as important as positive information to ensure a complete search.

_False____16. During a secondary search, speed is as critical as thoroughness.

_True____17. During the primary search, doors to rooms not involved in fire should be closed.

_False____18. To ensure a successful operation, firefighters must set aside concerns for their own safety while searching for victims.

_True___19. Firefighters should not kick a door to force it open.

_False____20. With a well-designed incident command or accountability system in place, firefighters can avoid becoming trapped or disoriented within a burning structure.