# Firefighting Equipment Terms and Definitions.

## A

- Adapter: plumbing accessories for connecting hoses and pipes of incompatible diameter, thread, or gender.

- Aerial Ladder: Permanently mounted power operated ladders with at least two sections and at least 50 ft total length.

- Attack hose: Fire hose used to apply water or other fire fighting agent directly to a fire or burning substance.

## B

- Bed Section: the non-extending section of an extension ladder.

## C

- Combination nozzle: A low pressure Fog Nozzle that can be adjusted to produce a near straight stream.

## D

- Delivery: Term for any hose that conveys water to a fire e.g., each outlet from a fire appliance supplies a delivery with water.

- Double Female Adaptor: fire hose adapter for connecting two “male” couplings together.

- Double Male Adaptor: fire hose adapter with two “male” connectors back-to-back; used for connecting two “female” couplings together.

## E

- Extension Ladder: A 20–60 foot ladder with one or more movable sections that extend beyond a base section, typically using a halyard rope and pulley mechanism for lifting and locking cams to latch the moving sections at a selected height.

## F

- Fog Nozzle: A nozzle that discharges water in small droplets. The droplets are unable to travel very far, but absorb heat very quickly because of the high surface area they present.

## G

- Green line: a garden hose.

- Ground ladder: A portable ladder designed to rest on the ground.

## H

- Halyard: utility rope for raising or lowering moving parts of extension ladder.

- Hook: forged steel hook at end of insulated pole of varying lengths; used for piercing and pulling building materials away from walls and ceilings.

- Hose: flexible conduit for moving liquids under pressure.

- Hose coupling: rigid interlocking end-pieces on fire hose; used for connecting hose to hydrants or fire engine pumps and other hose appliances

- Hose wrench: tool for holding hose couplings against opposite turning forces

## I

- Intake: Part of pump where water enters when pump forms partial vacuum.

## L

- Ladder Pipe: Nozzle attached to an aerial ladder and used to direct heavy stream from advantageous height.

- Life Safety Line: A rope used where its failure could result in serious injury; a rope used for connecting a firefighter/rescuer to a fixed anchor point or to another person.

## N

- Nozzle: A device attached to the end of a fire hose that directs, shapes, and regulates the flow of the water or fire fighting agent pumped into the hose. May have a control valve.

- Nozzle tip: Portion of a fire hose that forms the fire stream as it leaves the hose. Can be solid, fog, or other specialty nozzle

## P

- Penetrator Nozzle: A long narrow nozzle with a hardened steel tip designed to be forced through a wall or other obstruction to deliver water to a fire on the other side when other modes of access aren't possible or carry an unacceptable degree of risk.

- Pipeman: The firefighter who is on the nozzle attacking the fire.

- Pompier Ladder: A style of ladder that is also known as a "Scaling Ladder". It is used to climb from one window to another. It differs from other ladders in that it does not rest on the ground it instead uses a large hook at the top to attach to a window sill. The word "Pompier" is French for fireman.

- Pony Section: A shorter length of fire hose, especially large diameter hose, used to connect an apparatus to a hydrant or another apparatus.

## R

- Relief valve: A valve set to open at a specified pressure so as to not exceed safe operating pressure in hoses or pumps.

- Roof ladder: A single-section ladder with hooks on one end. The hooks are put over the ridge or peak of a roof to hold the ladder in place.

## S

## Siamese: Valve used to bring two smaller lines together into one larger line.

## T

- Turntable: rotating base of an aerial ladder that permits the ladder to be elevated and extended in any direction from a fixed location.

## U

- Utility rope: A rope not designed or maintained for life safety purposes.

## V

- Valve: mechanical means for stopping and starting flow in a conduit.

## W

- Water curtain nozzle: A nozzle designed to throw a fan of water droplets to form a curtain.

- Wye: hose coupling for splitting one line into two or more outlets.  Not to be confused with a Siamese which is used to bring two smaller lines together into one.