Textile Construction Methods

Unit Exam

Name ____________________________				FCS –TS-5

Date _____________________________				SEV5

Block ____________________________				NFCS 16.2

Matching: 			Loom Terms

A.	Woven			B.	Loom			C.	Weft

D.	Warp			E.	Shed			F.	Picking

G.	Beating up		H.	Take up		I.	Selvege

J.	Grain			K.	Bow			L	Skew

- _____	The intersection of warp and weft yarns.

- _____	The yarn transported across the opening of the shed during weaving.

- _____	A machine for weaving fabric.

- _____	The side edges of the woven fabric.

- _____	When the warp and weft yarns are off-grain and a curved line is formed in the

fabric.

- _____	The yarns that are wrapped onto the loom.

- _____	The yarns referred to as the filling yarns.

- _____	Winding finished fabric onto the fabric beam.

- _____	Pushing the filling yarn close against the woven fabric.

- _____	Term used to describe any fabric made on a loom.

- _____	The open area created on the warp yarns during weaving.

- _____	An off grain fabric when the filling yarn is at an angle other than 90 degrees to the

warp.

Matching:			Types of Looms

A.	Shuttle			B.	Shuttleless 		C.	Air-jet

D.	Multiphase		E.	Dobby			F.	Jacquard

G.	Rapier			H.	Water Jet		I.	Circular

J.	Triaxial

13.	_____	A loom that forms many different sheds at different places.

14.  _____	The type of loom that requires water to transport the filling yarn.

15.	_____	A loom that requires a shuttle.

16.	_____	The type of loom that utilizes a rod or steel tape to carry the filling yarn.

17.	_____	A loom that weaves complex patterned fabrics that originated in the Orient.

18.	_____	A loom where the filling yarn is guided through with a narrow jet of air.

19.	_____	A loom that weaves 3 sets of yarns producing fabrics that are stable in horizontal,

vertical and bias directions. Fabric is often used in hot air balloons.

20.	_____	A loom that produces fabrics for tubular fabrics

21.	_____	A loom that does not require a shuttle.

22.	_____	A loom that can produce terry cloth towels.

Matching:			Weaves and Fabric Quality

A.	Plain			B.	Basket			C.	Twill

D.	Satin			E.	Fabric Density		F.	Fabric Weight

G.	Fabric Hand		H.	Balance		I.	Abrasion Resistance

23.	_____	A weave that typically goes over two and under two warp yarns.

24.	_____	A characteristic used to describe the weight and thickness of textile products.

25.	_____	The simplest of the weaves.

26.	_____	Also referred to as the fabric count.

27.	_____	A weave that forms diagonal lines on the surface of the fabric.

28.	_____	The ratio of warp yarns to filling yarns in a fabric.

29.	_____	The ability of a fabric to resist wear.

30.	_____	The type of weave that forms a smooth surface by going over 4 or more warp

yarns at a time.

31.	_____	The way a fabric feels to the sense of touch.

True/False

32.	_____	Chiffon fabric is a plain weave fabric.

33.	_____	Batiste fabric is a satin weave fabric.

34.	_____	Oxford cloth is a twill weave fabric.

35.	_____	Herringbone fabric is a twill weave fabric.

36.	_____	Satin fabric is a satin weave fabric.

37.	_____	Percale fabric is a satin weave fabric.

38.	_____	Canvas fabric is a basket weave fabric.

39.	_____	Monk’s cloth is a twill weave fabric.

40.	_____	Muslin fabric is a plain weave fabric.

41.	_____	Houndstooth fabric is a basket weave fabric.

42.	_____	Chino fabric is a satin weave fabric.

43.	_____	Sateen fabric is a basket weave fabric.

44.	_____	Denim fabric is a twill weave fabric.

45.	_____	Sharkskin fabric is a basket weave fabric.

46. 	_____	Gingham fabric is a plain weave fabric.

Short Answer- In 3-5 sentences answer the following:

- How has technology changed the production of textiles in the last 100 years?

- What impact does textile construction have on the environment?