Electronics Test

Name:  _______Key_________________			Period: _____________________

What are three requirements for a circuit?

1. Source of energy

2. Load

3. Path

Explain what each band of a resistor means.

4. Band 1: write down the number the color represents

5. Band 2: write down the number the color represents

6. Band 3: write down the number of zero’s the color represents

7. Band 4: tolerance

The Space Machine Gun circuit has a 100k ohm potentiometer.  What are the readings when AB = 54K ohms?

- AC = ____100K______				9. BC = ____46K____

Give the colors or value for the following resistors.

10. Yellow/violet/black     _____47 ohm_______

11. 120K ohms ___brown/red/yellow_______

12. What does SPST (switch) stand for? Single pole single throw

13. Explain what happens to the current in a circuit when resistance of the same circuit increases or decreases? I increases when R deceases and I decreases when R increases

14. How much current could flow through a 36 ohm resistor in a car radio powered by a 12V battery?  I = E/R  		I= 12V/36 ohms

I = 1/3 amps

15. The current flow through a resistor is 0.3 A.  The measured voltage drop across the resistor is 600V.  What is the ohm value of the resistor?  R = E/I

R = 600V/.3A

R = 2000 ohms or 2K ohms

16. What voltage is needed for a circuit with a resistance of 1.2K ohms and a current of 3 Amps?    E = I x R

E = 3A x 1.2K

E = 3600V

17. Which cap will charge/discharge faster?

  - 100 micro farad

  - 120 micro farad

18. Circle the components that are not polarity sensitive.

Cap              Resistor	LED			Photocell

19. Why does a technician use a “pot” while designing a circuit?

  - To check a circuits voltage

  - To vary the resistance for exact I or E results the circuit needs.

20. Why did the LED stay on for several seconds after the power was disconnected from the cap circuit?  The capacitor stores electrons

21. A resistors job is to ___oppose___ current?

22. Why does a designer use a resistor as a fuse? Resistors are very cheap

Binary to Decimal ---- Decimal to Binary

(23) 1101 = 13		(24)	12 = 1100        (25) 10011 = 19	  (26)	21 = 10101

27. Device that is triggered by placing voltage on its gate 	 SCR

28. Provides a simple way to open and close a circuit.  Switch

29. Changes its resistance when hit by light photocell

30. Two or more cells battery

31. Opposition to the flow of current resistance

32. Electromotive force or pressure required moving the electrons through the circuit voltage

33. Intensity flow of electrons through a conductor current

34. Resistance measurement ohms

35. Current measurement Amps

36. “Cap” measurement microfarad

37. Basic equation to make an electronic circuit balanced ohms law

38. How is E measured using a Multi-meter?

  - Break the circuit

  - Across the component

39. How is I measured using a Multi-meter?

  - Break the circuit

  - Across the component

Name two advantages caused by placing the LED’s of the mask circuit in parallel.

40. one LED burns out the others will still work

41. the E will be the same on all 4 LED’s

Place a “C” by each conductor and an “I” by each insulator:

42: _I_ sponge						43: __C_ resistor

44. _C___ wire						45: __C_ paper clip

46. _I__ dielectric